# A10 - Reliability and Safety Analysis

# Year: 2025	Semester: Spring	Team: 9     Project: Fairway Finder

# Creation Date: March 31st, 2025 		Last Modified: October 20, 2023

# Author:  John Stanwick	Email: jstanwic@purdue.edu

# Assignment Evaluation: See the Rubric in the Brightspace Assignment

# Reliability Analysis

# For this reliability and safety analysis, we chose to analyze the following components: STM32F411xC Microcontroller, ICM-20948 9-Axis Sensor, AP63205 Buck Converter, and the LM1117 Low Dropout Regulator. Two of these components, the microcontroller and 9-axis sensor, are our most complex IC implementations. While all of the components that we included in our design are critical to the fundamental operation of the robot, these two components contribute the most to the algorithmic operation of the robot. Likewise, the other two components, the AP63205 and the LM1117, are selected due to being the two components in our design that are most likely to fail. These two components handle the highest load from the battery to the motors.

# Of principle concern is the failures per million hours model, which is given in the MIL-HDBK-217F Handbook [4] as:

# λP = (C1 πT + C2 πE) πQ πL

# Equation 1

# This is the expressed multiplication of the die complexity failure rate, temperature coefficient, and the package failure rate, environmental factor, all multiplied by the quality factor and the learning factor. Once the failures per million hours is determine, it becomes relatively trivial to calculate the mean time to failure, which is simply:

# MTTF = 10^6 / (24 * 365 * λP) years

# Equation 2

# This equation is intuitive because it is simply the Mean Time Between Failures rate multiplied by 10^6 hours. These equations and any subsequent calculations are given by the MIL-HDBK-217f Military Handbook.

# The component quality factor, or πQ of all components used in this analysis is 10, or the commercial screening level coefficient. The newest component that we have is the ICM-20948, which was released in early 2024, and thus is awarded a πL of 1.5. All other components are >2 years from manufacturing date and thus are awarded a πL of 1. Our components must operate grounded in motion and thus are awarded a πE of 4. The other parameters vary between components and are outlined in their respective tables.

# STM32F411xC Microcontroller [5]

| # Parameter Name | # Description | # Value | # Comments |
| --- | --- | --- | --- |
| # C1 | # Die Complexity Failure Rate | # 0.56 | # MOS, 32 Bit Microprocessor |
| # πT | # Temperature Coefficient | # 0.98 | # MOS, Operable up to 85*C |
| # C2 | # Package Failure Rate | # 0.053 | # Contains 81 pins |
| # πE | # Environmental Factor | # 4 | # Grounded, Mobile Environment |
| # πQ | # Quality Factor | # 10 | # Commercial Part |
| # πL | # Learning Factor | # 1 | # > 2 Years in Production |
| # λP | # Failures Per Million Hours | # 7.608 | # Equation 1 |
| # MTTF | # Mean Time To Failure | # 15.00 Years | # Equation 2 |

# ICM-20948 9-Axis Sensor [2]

| # Parameter Name | # Description | # Value | # Comments |
| --- | --- | --- | --- |
| # C1 | # Die Complexity Failure Rate | # 0.28 | # MOS, 16 Bit Microprocessor (Onboard) |
| # πT | # Temperature Coefficient | # 0.98 | # MOS, Operable up to 85*C |
| # C2 | # Package Failure Rate | # 0.0087 | # Contains 24 pins |
| # πE | # Environmental Factor | # 4 | # Grounded, Mobile Environment |
| # πQ | # Quality Factor | # 10 | # Commercial Part |
| # πL | # Learning Factor | # 1.5 | # 1 Year in Production |
| # λP | # Failures Per Million Hours | # 4.638 | # Equation 1 |
| # MTTF | # Mean Time To Failure | # 24.61 Years | # Equation 2 |

# AP63205 Buck Converter [1]

| # Parameter Name | # Description | # Value | # Comments |
| --- | --- | --- | --- |
| # C1 | # Die Complexity Failure Rate | # 0.02 | # 101 to 300 Bipolar Transistors |
| # πT | # Temperature Coefficient | # 7.0 | # MOS, Operable up to 160* C |
| # C2 | # Package Failure Rate | # 0.0019 | # Contains 6 Pins |
| # πE | # Environmental Factor | # 4 | # Grounded, Mobile Environment |
| # πQ | # Quality Factor | # 10 | # Commercial Part |
| # πL | # Learning Factor | # 1 | # > 2 Years in Production |
| # λP | # Failures Per Million Hours | # 1.476 | # Equation 1 |
| # MTTF | # Mean Time To Failure | # 77.34 Years | # Equation 2 |

# LM1117 Low Dropout Regulator [3]

| # Parameter Name | # Description | # Value | # Comments |
| --- | --- | --- | --- |
| # C1 | # Die Complexity Failure Rate | # 0.02 | # 101 to 300 Bipolar Transistors |
| # πT | # Temperature Coefficient | # 50 | # Linear, Operable up to 125* C |
| # C2 | # Package Failure Rate | # 0.0026 | # Contains 8 Pins |
| # πE | # Environmental Factor | # 4 | # Grounded, Mobile Environment |
| # πQ | # Quality Factor | # 10 | # Commercial Part |
| # πL | # Learning Factor | # 1 | # > 2 Years in Production |
| # λP | # Failures Per Million Hours | # 10.104 | # Equation 1 |
| # MTTF | # Mean Time To Failure | # 11.30 Years | # Equation 2 |

# While all selected components have acceptable mean time to failure values for the intended application, the LM1117 LDO regulator stands out as the most vulnerable with a mean time to failure of only 11.30 years, largely due to its high temperature coefficient. Given that the microcontroller and sensor are essential for the robot’s algorithmic operations and the power components are most likely to fail due to high load conditions, improvements should be made in enhancing the thermal management of the power circuits and possibly selecting regulators with improved temperature ratings. Further, military grade parts would yield a significant increase in mean time to failure due to the nature of the model used. Refinements such as implementing redundant power pathways and incorporating more real-world environmental testing could further boost the robustness and reliability of our circuit design.

# Failure Mode, Effects, and Criticality Analysis (FMECA)

# There are 3 criticality levels for the fairway finder. Low, Medium, and High. Low criticality is defined as any failure that poses no safety concern or little long-term functionality concern. An example would be any problem that is fixed by simply power cycling the robot. The acceptable failure rate for low concern failures is l < 10-5. Medium criticality is defined as any failure that poses low safety concern or some amount of long-term functionality concern. An example would be a problem with movement operation. The acceptable failure rate for medium concern failures is l < 10-7. High criticality is defined as any failure that poses significant safety concerns or significantly impacts the long-term functionality of the robot. The acceptable failure rate for high concern failures is l < 10-9.

# For this analysis, all environments are assumed to be sunny, 75*F, and dry conditions. This is the ideal condition for our robot and a likely possibility for consistent robot operation. Beyond that, we assume that there is no debris in the robot and the user is operating the robot as intended.

# 3.0 Sources Cited:

[1]“AP63200/AP63201/AP63203/AP63205 Datasheet,” Nov. 2024. Available: 	https://www.diodes.com/datasheet/download/AP63200-AP63201-AP63203-AP63205.pdf (accessed Apr. 03, 2025).

[2]“ICM-20948 Datasheet,” 2024. Available: https://invensense.tdk.com/wp-content/uploads/2024/03/DS-000189-ICM-20948-v1.6.pdf (accessed Apr. 03, 2025)

[3]“LM1117 800-mA, Low-Dropout Linear Regulator.” Jan. 2024. Available: https://www.ti.com/lit/ds/symlink/lm1117.pdf (accessed Apr. 03, 2025)

[4]“Military Handbook Reliability Prediction of Electronic Equipment,” Jan. 02, 1990. Available: https://purdue.brightspace.com/d2l/common/viewFile.d2lfile/Database/NDQyNzQwNzI/Mil-Hdbk-217F.pdf?ou=1216648 (accessed Apr. 03, 2025).

‌

[5]“STM32F411xC STM32F411xE Datasheet,” 2017. Available: https://www.st.com/resource/en/datasheet/stm32f411ce.pdf (accessed Apr. 03, 2025)

# Appendix A:  Schematic Functional Blocks

# Subsystem A: Power Circuits

Schematic A.1: AP63205 Buck Converter

Schematic A.2: LM1117 Low Dropout Regulator

# Schematic A.3: Battery Connector

# Subsystem B: Sensor Block

# Schematic B.1: HM-10 Bluetooth Module

# Schematic B.2: SAM-M8Q GNSS Module

Schematic B.3: ICM-20948 9-Axis Module

# Subsystem C: Motor Block

Schematic C.1: DRV8871DDAR Motor Driver 1

# Schematic C.2: DRV8871DDAR Motor Driver 2

# Subsystem D: Microcontroller Block

Schematic D.1: STM32F411XC Microcontroller

# Appendix B:  FMECA Worksheet

# Subsystem A: Power Circuits

| Failure No. | Failure Mode | Possible Causes | Failure Effects | Method of Detection | Criticality | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Battery converter voltage too high | AP63205 Buck Malfunction | Overheating of circuits Smoking of circuits | Immediate failure of multiple subsystems | High | Catastrophic failure. Potential to brick all systems |
| 2 | Battery converter voltage too low | AP63205 Buck Malfunction, Low battery | Missed data transmission | Consistent failure of specific subsystem (likely ICM-20948 as it is lowest voltage component) | Low | This failure will happen due to the battery voltage being variable. We need to make sure our voltage tolerances are high enough to compensate |
| 3 | Short circuit | All components | Overheating of circuits Smoking of circuits | Immediate failure of one specific subsystem | High | Targeted catastrophic failure. Potential to brick influenced subsystem |

# Subsystem B: Sensor Block

| Failure No. | Failure Mode | Possible Causes | Failure Effects | Method of Detection | Criticality | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | Bluetooth transmission failure | HM-10 software bug, interference in Bluetooth frequencies | Bluetooth commands will become unintelligible | Loss of control from app | Medium | Low risk to user. Shut down button is in place to mitigate this |
| 5 | GPS transmission failure | SAM-M8Q software bug, bad reading from a specific fix | Transmitted microcontroller GPS coordinates will become inaccurate | Robot not changing direction / speed while in follow me mode | Medium | Low risk to user. All unreasonable coordinates are automatically disqualified from follow-me algorithm |
| 6 | 9-Axis sensor transmission failure | ICM-20948 software bug, magnet nearby | Robot direction and movement miscalibration | Robot making sporadic movements and not turning to right fix | Medium | Low risk to user. If the magnetometer is out of alignment, then the robot will simply not turn to the right direction |

# Subsystem C: Motor Block

| Failure No. | Failure Mode | Possible Causes | Failure Effects | Method of Detection | Criticality | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 7 | Motor driver failure | Overcurrent, overheating, rotation blocking | Loss of motor movement | Robot will stop moving | High | Motor driver failure has long-term performance ramifications. If the motor fails, it may require a complete replacement |
| 8 | Motor block short circuit | Manufacturing defect, water presence | Complete failure of motor block | Robot will stop moving | High | A motor short circuit could cause catastrophic failure due to the high voltage / currents used. |

# Subsystem D: Microcontroller Block

| Failure No. | Failure Mode | Possible Causes | Failure Effects | Method of Detection | Criticality | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 9 | Microcontroller Freeze / Crash | Software bug, memory corruption | Total loss of control | Watchdog timer, LED health monitoring | High | Risk to the user is low but causes immense long-term ramifications. |
| 10 | Microcontroller overheating | Lack of proper thermal management | Reduced performance | Follow-me algorithm reduced fidelity and / or motor control is delayed | Low | We need to make sure proper cooling is in the robot because of the sunny environment |
| 11 | Firmware corruption | Power cycling microcontroller too fast / with too much current | Complete loss of control and ability to regain control | Firmware checksums, error logging, watchdog timer | High | Potential for catastrophic failure, especially in the field |