# Software Formalization

# Year: 2025

# Semester: Spring

# Team: 9

# Project: Fairway Finder

# Creation Date: February 18th, 2025

# Last Modified: September 15, 2023

# Author:  John Stanwick

# Email: jstanwic@purdue.edu

# Assignment Evaluation: See Rubric on Brightspace Assignment

# 1.0 Utilization of Third Party Software

| # Software | # Source | # Description | # Usage | # Licensing |
| --- | --- | --- | --- | --- |
| # STM32 HAL Library | # https://www.st.com/resource/en/user_manual/um1725-description-of-stm32f4-hal-and-lowlayer-drivers-stmicroelectronics.pdf | # Hardware Abstraction Layer (HAL) for STM32 microcontrollers | # Allows for easy-to-use drivers and linking software | # Open-Source, Free for non-commercial use |
| # u-blox GNSS Library | # https://www.u-blox.com/en/product/ubxlib | # Provides example communication and data parsing for the SAM-M8Q GPS module. | # Used as reference code for obtaining and processing the GNSS position data | # Open-Source, Free for non-commercial use |
| # Bluefruit LE Connect Library | https://github.com/adafruit/Bluefruit_LE_Connect_v2 | # Bluetooth communication examples and standards | # Used for debugging, validating, and transferring BLE signals to the HM-10 Module | # Open-Source, Free for all use |
| # Apple Developer Program | # https://developer.apple.com/programs/ | # Apple’s app developer program | # Used to develop and publish apps on the app store and TestFlight | # $99 / yr |

# 2.0 Description of Software Components

| # Component | # Type | # Description | # Development Strategy |
| --- | --- | --- | --- |
| # Microcontroller Firmware | # Embedded | # Controls core operations, including motor control, GNSS processing, Bluetooth communication, and power management | # Fully developed by the team using STM32 HAL libraries |
| # Motor Control | # Embedded | # Uses PWM to control motor speed and direction, integrates PID control for smooth movement | # Developed using STM32 HAL Motor Controller abstractions |
| # GNSS Data Processing | # Embedded | # Parses NMEA sentences from the SAM-M8Q module to extract location data | # Partially ported from u-blox GNSS example code. Custom parsing and filtering developed in-house |
| # Bluetooth Communication | # Embedded | # Manages Bluetooth Low Energy (BLE) communication between the robot and the mobile app | # Uses BLE communication examples as reference but implements a custom protocol for data exchange |
| # Follow-Me Algorithm | # Joint Mobile and Embedded Algorithm | # Processes GNSS data to determine user position relative to the robot and activates motor control to follow that user | # Developed in house using mobile and robotic GNSS data |
| # Mobile App | # Mobile App | # Provides a user interface for monitoring robot status, sending commands, and viewing GNSS data | # Developed in-house using Expo, React Native, and Apple’s development tools |
| # Power Management System | # Embedded | # Monitors battery voltage and current consumption to optimize power usage | # Developed using HAL power monitoring |

# 3.0 Testing Plan

| # Component | # Testing Description | # Priority (1 – 5, 5 is high) |
| --- | --- | --- |
| # Microcontroller Firmware | # Verify that the firmware initializes correctly, runs tasks as expected, and does not crash under load. Test UART communication, task scheduling, and interrupt handling. | # 5 |
| # Motor Control | # Test PWM signal accuracy, ensure proper PID control operation, and verify that the motor responds correctly to speed and direction commands. Ensure no excessive jitter or lag in motor responses. | # 5 |
| # GNSS Data Processing | # Validate NMEA sentence parsing accuracy, check for errors in coordinate extraction, and compare output to known test locations. Test GNSS signal reception and update rates. | # 4 |
| # Bluetooth Communication | # Ensure stable BLE connection, validate data packet integrity, and test transmission/reception latency. Confirm that commands from the mobile app are correctly interpreted by the microcontroller. | # 4 |
| # Follow-Me Algorithm | # Conduct real-world movement tests to verify that the robot correctly follows the user within a reasonable error margin. Simulate GPS drift conditions to assess response accuracy. | # 4 |
| # Mobile App | # Verify UX functionality, BLE connection stability, GNSS data display, and command transmission. Test responsiveness under different mobile OS versions. | # 3 |
| # Power Management System | # Measure battery consumption under different loads, verify voltage regulation, and check low-battery detection accuracy. Ensure power-saving features work correctly. | # 2 |

# 4.0 Sources Cited:

[1] adafruit, “GitHub - adafruit/Bluefruit_LE_Connect_v2: iOS app for use with Bluefruit Bluetooth LE modules and dev boards from Adafruit (v2.0),” GitHub, 2016. https://github.com/adafruit/Bluefruit_LE_Connect_v2 (accessed Feb. 22, 2025).

[2] Apple, “Apple Developer Program - Apple Developer,” Apple.com, 2019. https://developer.apple.com/programs/

[3] “Introduction Description of STM32F4 HAL and low-layer drivers UM1725 User manual,” 2021. Available: https://www.st.com/resource/en/user_manual/um1725-description-of-stm32f4-hal-and-lowlayer-drivers-stmicroelectronics.pdf

# ‌

[4] “ubxlib: u-blox host library,” u-blox, Jun. 28, 2022. https://www.u-blox.com/en/product/ubxlib (accessed Feb. 22, 2025).

‌

‌

# Appendix 1: Software Component Diagram

# Diagram 1: Software Component Flowchart