Team Information

- Section: ___________________________

- Date: ___________________________

Team Members (write full names):

- ____________________________________________

- ____________________________________________

- ____________________________________________

- ____________________________________________

(Optional) Team Name: ___________________________________________

Instructions

This in-class activity guides you through creating a MATLAB script that reads the X and Y voltages from a Grove Thumb Joystick connected to an Arduino Uno R4 WiFi and uses loops (for or while) to visualize its motion in real time. You will generate a trajectory plot to show how the joystick position changes over time. - Complete all work in MATLAB and insert your circuit diagram, MATLAB code, trajectory plot, and Command Window output directly into this Word file. - Save and submit both your Word file and MATLAB script through the designated Brightspace In-Class Activity Drop Box. - File naming convention: - Word file → ENGR132_ICA_Loops_Joystick_LastNames.docx - MATLAB script → ICA132_Loops_Joystick_LastNames.m - MATLAB figure → ENGR132_ICA_Loops_Joystick_LastNames.png

Relevant Course Resources:

| Pre-Class Videos | - None |
| --- | --- |
| Course Resources | - Getting Started with MATLAB and Arduino Support Package - MathWorks Example: Read Analog Input and Plot Sensor Data in Real Time – Demonstrates how to read analog voltages continuously and visualize them in MATLAB using loops. - Seeed Studio Grove Thumb Joystick Wiki. Technical overview, pin mapping, and wiring guide. - ENGR 132 MATLAB Programming Standards page (on Brightspace) |

Submission Instructions:  Re-name your files as: • Word file → ENGR132_ICA_Loops_Joystick_LastNames.docx • MATLAB script → ICA132_Loops_Joystick_LastNames.m • MATLAB figure → ENGR132_ICA_ Loops_Joystick_LastNames.png  Submit all files through the designated Brightspace In-Class Activity Drop Box at https://purdue.brightspace.com/.  Ensure that all team members are listed in the submission on Brightspace (if applicable) and that everyone reviews and approves the submission.

## Task 1: Circuit Setup

  - Assemble the circuit as shown in Figure 1.

Connect the Grove Thumb Joystick directly to the A0 port on the Grove Base Shield, as shown in the image below.

Figure 1: Grove Thumb Joystick connected to the A0 port on the Grove Base Shield. The A0 port internally connects to two analog pins on the Arduino Uno R4 WiFi: A0 → X-axis (horizontal movement), A1 → Y-axis (vertical movement).

Ensure that the Grove Base Shield is firmly seated on the Arduino Uno R4 WiFi and the USB cable is connected to your computer.

## Task 2: MATLAB Script Completion (Loops and Joystick Motion Plot)

In this task, you are provided with a partially completed MATLAB script that includes pseudocode comments to guide your work. Your job is to fill in the missing lines of code so that the program runs correctly and produces a real-time plot of the joystick motion.

Follow the structure shown in the code provided below, and use what you have learned about for loops, data acquisition, and plotting.

You will:

- Connect the Arduino Uno R4 WiFi and set up the Grove Thumb Joystick on port A0.

- Use a for loop to repeatedly read the joystick voltages (X and Y).

- Store and plot the readings in real time to show the joystick trajectory (X vs Y).

- Display summary information (average voltages) in the Command Window.

Complete the MATLAB script below by filling in the missing code sections.

## Partial MATLAB Code (with Pseudocode Guidance)

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

% ENGR 132 - ICA: Joystick Motion Plot

% Authors: _________________________

% Date: ____________________________

%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%%

%% ---------------------------------------------------------------

%% 1) Connect to Arduino

% Pseudocode:

%   - Clear workspace and Command Window

%   - Connect Arduino Uno to MATLAB

clear; clc; close all;

a = ___________________________;     % connect Arduino Uno

%% ---------------------------------------------------------------

%% 2) Set up Grove Thumb Joystick

% Pseudocode:

%   - Configure analog input pins for X and Y axes

%   - Reference voltage is 5 V for the Uno

configurePin(a,'A0','AnalogInput');   % X-axis

configurePin(a,'A1','AnalogInput');   % Y-axis

VREF = 5.0;

%% ---------------------------------------------------------------

%% 3) Define sampling parameters

% Pseudocode:

%   - Choose N = number of samples

%   - Choose Ts = time delay between readings

%   - Create empty vectors xData and yData for data storage

N  = _______;        % number of samples

Ts = _______;        % delay between readings (s)

xData = _______;

yData = _______;

%% ---------------------------------------------------------------

%% 4) Prepare figure

% Pseudocode:

%   - Open figure window

%   - Set axis limits centered at (0,0)

%   - Add grid, labels, and title

figure('Name','Joystick Motion Plot');

axis([0 VREF 0 VREF]); grid on; box on;

xlabel('X Axis (V - centered)');

ylabel('Y Axis (V - centered)');

title('Joystick XY Trajectory - Arduino Uno');

%% ---------------------------------------------------------------

%% 5) Data collection loop

% Pseudocode:

%   FOR each sample from 1 to N:

%       - Read X voltage from A0 and subtract 2.5

%       - Read Y voltage from A1 and subtract 2.5

%       - Store values in xData and yData

%       - Plot all points collected so far (blue line)

%       - Plot current point (red dot)

%       - Pause for Ts seconds

for k = 1:N

xData(k) = ________________________;   % read and center X

yData(k) = ________________________;   % read and center Y

plot(xData(1:k), yData(1:k), 'b.-', 'LineWidth', 1.2);

hold on

plot(xData(k), yData(k), 'ro', 'MarkerFaceColor','r');

hold off

axis([0 VREF 0 VREF]); grid on;

pause(Ts);        % use pause instead of drawnow

end

%% ---------------------------------------------------------------

%% 6) Display summary results

% Pseudocode:

%   - Print number of samples

%   - Calculate and display average X and Y voltages

fprintf('Samples collected: %d\n', N);

fprintf('Average X voltage: %.3f V\n', _______________________);

fprintf('Average Y voltage: %.3f V\n', _______________________);

%% ---------------------------------------------------------------

%% Tips

% • Suggested values: N = 100, Ts = 0.05.

% • Only one Grove cable connected to port A0 is required (it carries both X and Y signals).

% • The center position corresponds approximately to 2.5 V on each axis.

Tips:

- Suggested values: N = 150, Ts = 0.2.

- Only one Grove cable connected to port A0 is required — it carries both X and Y signals.

Solution

## Task 3: Plot and Output Documentation

After running your completed MATLAB script:

In the box below, capture your plot.

## Plot:

## Task 4: Command Window Output

Include a screenshot of your MATLAB Command Window showing the number of samples collected and the average X and Y voltages, and paste it in the box below.

## Answer:

## Task 5:  Reflection

In 1–2 sentences, describe how the plotted trajectory changes as you move the joystick and what the average voltages tell you about its center position.

## Answer:

Communication Errors between the Arduino and MATLAB: - Check the USB connection. Make sure your Arduino Uno is firmly connected using the correct USB cable (USB-A to USB-B). A green power LED should turn on when connected properly. - Verify the COM port. Run seriallist in MATLAB to identify the correct COM port. Use that port in your connection command (e.g., arduino('COM5','Uno')). - Install Arduino drivers. Ensure all required drivers are installed. Reinstall the MATLAB Arduino Support Package if needed (Home → Add-Ons → Get Hardware Support Packages). - Restart MATLAB. Close and reopen MATLAB if the connection fails. - Restart your computer. Sometimes a full restart resolves USB or COM port conflicts.