LAB. #3,  MACHINING EXPERIMENT IPRIVATE 

STRAIGHT AND FACE TURNING 
Purpose:    To study methods for setting up and operating a lathe, and the use of measuring instruments to produce accurate workpiece dimensions.

Machine:   Engine lathe 

Tools:      6 inch dial caliper, steel ruler, 


     uncoated carbide tools

Material:  Aluminum 6061-T6 bar (approximately 2 inch in diameter)

Operations:
A.  Determine the relationship between cross slide and cross slide dial.

    1.
Select any point on the cross slide.  Turn cross slide crank 0.25 in.  Measure the distance the cross slide moves.

    2.
If cross slide moves 1/8", the dial reads direct on the diameter.

    3.
If cross slide moves 1/4", the dial reads direct on the radius, or depth of cut.

C. Set up tool.

   1. 
Place the carbide insert in a right-hand tool holder.  Tighten finger tight.

   2. 
Insert tool holder onto tool post, adjust tool bit at approximate right angle to axis of centers.  See Figure #1. 

   3. 
Set the height of the tool to the same height as tailstock center point (slightly below is better than slightly above center).  See Figure #1.  Safety Note: Point of tool is sharp, be careful when inserting workpiece.

D. Insert workpiece.

  1. 
Insert the workpiece into the chuck.

  2. 
Tighten the three jaw chuck.

E. Set spindle speed.

  1. 
Calculate RPM from cutting speed and diameter (in inches) of the workpiece.  For specific cutting speeds, see steps F-1, G-1, and H-1 as you perform each operation.




     V x 12

V x 4



RPM = -----------------  ( ---------------  




    π x D.  
  
    D


D: diameter in inches,
V: cutting speed in ft/min.

  2. 
Locate closest RPM on spindle speed chart, located on headstock of lathe.  Slide the handle under that column.  Note: Spindle needs to be rotated to make sure that gears are properly engaged.

   3.
Locate feed on the thread and feed chart, using the F column.  Using the four handles and/or dials, set it accordingly.  Note: Spindle needs to be rotated to make sure that gears are properly engaged.

F. Facing of workpiece.

  1. 
Place an insert in a Right‑Hand tool holder.

  2.
Set RPM for a cutting speed of 400 ft/min.

  3. 
Set power cross feed to 0.0154".

  4.
Ensure tool height is set correctly.

  5. 
Set direction of cross slide travel towards the center of the workpiece.

  6.
Turn the lathe on with control lever and advance your tool until it touches the end of the workpiece.  This is your reference point. 

  7.
Crank the cross slide towards you until your tool clears the workpiece.

  7.
Engage the cross feed to make a trial facing cut in center of workpiece.  Reset point of the tool to the center of projection, if the circle cut appears asymmetrically positioned on the face of the workpiece.  Set the digital readout to zero (Z direction).  

  8.
Set depth of cut to 0.020" by moving the carriage (z axis).

  9.
Turn lathe on and engage power cross feed.  After facing to the center of the workpiece, disengage power feed.

10.
Reset digital readout to zero.  This will be your z axis zero for the remaining cuts.

11. Move carriage away from the part, then crank the cross slide out towards you until tool clears the workpiece and turn the lathe off.   

G.   Rough turning

1. Set RPM for a cutting speed of 450 ft/min.  Refer back to steps E-1 and E-2.

2. Set feed for 0.007 in./revolution.  Refer back to step E-3.

3. Turn the lathe on by pulling control lever up and advance cross slide until tool bit touches rotating workpiece.  This is your reference point for setting the depth of cut.  Zero digital readout.  

4. Take a light cut of 0.025” or so to your first z step.   Move the carriage away from the part without changing the x-axis setting.   Stop the machine and measure the part diameter.   Now enter this dimension for x on the digital readout.

5. Advance tool with cross slide crank to remove 0.250" from diameter of workpiece (use multiple cuts of 0.100” or less).

6. Turn lathe on.  Engage power longitudinal feed.  Turn the initial 0.750 inches only.  Disengage power feed.  Return carriage to starting position without backing the tool from the workpiece.  Turn lathe off.

H.  Semi-finish turning

    1.
Set RPM for a cutting speed of 500 ft/min.

    2.
Feed remains the same as in rough turning.

    3.
Measure the diameter of the workpiece and set the depth of cut such that the diameter after semi-finish turning is about 1.630 inches.

   4.
Turn approximately 1/4" in length.  Stop and check the diameter.  If correction is needed, return carriage to starting position and make correction.

   5.
Repeat step G-6. 

I.  Finish turning

   1.
RPM remains the same as for semi-finish turning.

   2.
Set feed for 0.003 in./revolution.

   3.
Measure the diameter and adjust the cross slide to produce a diameter of 1.600" ± 0.002"

   4.
Repeat step H-4

   5.
Repeat step G-6.

J.  Completion of the part

  1.
Repeat steps H and I for the other diameter.

  2.
Ask the lab instructor to show you how to add the taper.

K.  Clean machine and remove chips from pan and floor.  Safety Note:  If necessary, remove tool bit from tool holder first, turn the lever away from you, and then remove tool holder from tool post.

L.  Return tools to cabinet and workpiece to instructor.

Computation
1.
Cutting conditions

2.
Ideal cutting time

3.
Material removal rate

Measurement
1. Final diameter of the workpiece

2. Lengths of the cut sections from the end (faced surface).

Straight Turning OperationPRIVATE 

[image: image1.jpg]



Figure 1:  Straight turning set-up

(a)
Roughing Cut

    (b)
Semifinishing Cut

(c)
Finishing Cut

V = ________ fpm

V = ________ fpm


V = ________ fpm


N = ________ rpm

N = ________ rpm


N = ________ rpm


f = _________ ipr

f = _________ ipr


f = _________ ipr


d = _________ in

d = _________ in


d = _________ in


l = _________ in


l = _________ in


l = _________ in

[image: image2.png]~—0.750

1673 —=

E!





Figure 2:  Desired geometry of the finished part.










