ME 576 Laboratory Assignment 			Session 7

Generate PLC programs and generate the truth table to verify the results.   Store the programs in a USB 2.0 flash drive and simulate them on the NUM PLC.

1)

E0

E1	        E3          A0

E2

2)	Write a short program to perform the following operation according to two inputs I0 and I1.

operation: 79  85   (Send output to AN2)

I0 (E1)		0 	0	1	1

I1 (E2)		0	1	0	1

		and	or	nand	xor

3)

E0      E1

E2		      A8

E3      E4

A9

E5

4)	Operation: Motor control problem

Motor = (Motor + start) AND stop AND clamp1 AND clamp2

Use the following variables:

Motor: A0

Start: E1

Stop: E2

Clamp1: E3

Clamp2: E4

Procedures for simulation on NUM machine:

1 -  Open the ME576 project template

Start Flexium Tools software in NUM machine. Under “Getting started”, click on “Open a project from disk”. Select “ME576PLC.project”. The template has been designed for pre-mapping of the variables to the correct addresses of the PLC numpad.

Click on Finish to accept all the default NUM project configurations

2 – Start programming in MainPRG under PLC Logic → Application

Refers to the CodeSys Manual for instructions on how to use the built in function.

3 – Convert the program from instruction list to ladder diagram to verify your code.

4 – Simulate the PLC program using NUM numpad buttons: