Homework 9:  Software Design Considerations

Due: Friday, October 26, at NOON

Team Code Name: Hooked on Harmonix                                                               Group No.  2

Team Member Completing This Homework: Tom Bottonari

e-mail Address of Team Member: tbottona@ purdue.edu

NOTE:  This is the last in a series of four “design component” homework assignments, each of which is to be completed by one team member.  The completed homework will count for 20% of the individual component of the team member’s grade.  The body of the report should be 3-5 pages, not including this cover sheet, references, attachments or appendices.

Evaluation:

| ## SCORE | ### DESCRIPTION |
| --- | --- |
| 10 | # Excellent – among the best papers submitted for this assignment. Very few corrections needed for version submitted in Final Report. |
| 9 | # Very good – all requirements aptly met. Minor additions/corrections needed for version submitted in Final Report. |
| 8 | # Good – all requirements considered and addressed. Several noteworthy additions/corrections needed for version submitted in Final Report. |
| 7 | # Average – all requirements basically met, but some revisions in content should be made for the version submitted in the Final Report. |
| 6 | # Marginal – all requirements met at a nominal level. Significant revisions in content should be made for the version submitted in the Final Report. |
| * | # Below the passing threshold – major revisions required to meet report requirements at a nominal level. Revise and resubmit. |

* Resubmissions are due within one week of the date of return, and will be awarded a score of “6” provided all report requirements have been met at a nominal level.

Comments:

Comments from the grader will be inserted here.

- Introduction

Our project, formally known as "Hooked on Harmonix", provides both an educational and entertaining experience for those with aspirations of playing the piano.  This game will store a list of songs stored in MIDI format (about 5 for the purposes of our project) that the user will be prompted to select from.  The top-level design of our software will be modeled like a state machine. Upon boot, the start screen will be loaded into the video buffer (controlled by an FPGA – Field Programmable Gate Array) displaying the text "press any key to continue" as well as our project name. After a key is pressed, the song selection screen will be loaded into the video buffer, displaying the currently available songs to play. Once the user chooses one of the available songs, the main gaming screen is loaded (mostly black screen with keyboard at the bottom, and scoring text just below it). The software will remain in this state for most of the user interaction with the game. Within this gaming state, our code will be interrupt driven. The processor will parse the selected MIDI file and obtain the timing information for the song and then sleep until either a note-pressed event has occurred, or it is time to send out timing information to the FPGA based on the parsed MIDI file. This whole time, in hardware, the FPGA is deciding which data gets put into the video buffer, as well as managing how it is output to the computer monitor via VGA interface. In order to eliminate artifacts on the screen, we are using a method of double buffering called "page-flipping", which will be described later. Once the song is complete, our program then moves into the final state of our state machine, which is just a screen that displays the user's performance statistics. Once a key is pressed in this state, it returns back to the song selection page and begins the process over again. We will have an asynchronous reset button on our box that will act as a "soft" reset, and restore the program back to the startup page. Any "hard" resets will be taken care of by simply power cycling.

- Software Design Considerations

When designing a standalone videogame of any kind, it is integral to obtain a robust software design due to the fact that in addition to receiving external input and responding accordingly, visual output can easily become a headache. In order to achieve success with these tasks, it is very important that we break all of them into different modules to avoid as much confusion as possible. From a top-level perspective, the tasks of handling external input and computations vs. visual output have been separated into different programs. As previously mentioned, the Cyclone II FPGA will handle the VGA output and video buffering, while the Nios II embedded processor will parse the MIDI file and handle all MIDI input as well as audio output. It is for this reason that our software design is quite unique from any other group. Early on, we decided that we would like the screen to be as large as possible, so that it was easier for the user to translate the scrolling blocks on the screen to the appropriate key. This quickly ruled out using an LCD, so we decided to go with a VGA interface. However, it quickly became apparent that the any normal processor would not be sufficient to achieve video output via VGA interface due to the fact that it must always be sending out pixel information, thus using up all of its processing power. However, this can be achieved through hardware circuits. Thus, we are using an FPGA to handle our VGA output, and although this is technically a hardware implementation, it will be discussed further in this document, since it still involves coding. As you can see, from the start it was important to have a clear separation between which coding style will handle which task, and communication between the two programs is very essential. The further sub-modules of these two different coding styles will be further described in section 3.0.

The Altera Cyclone II FPGA that we are employing in our design contains about 239 Kb of on-chip programmable RAM/ROM memory in what is referred to as the EPCS Controller Core (as seen in Figure 1 below) [1]. However, the primary configuration device is an external serial flash chip called the EPCS Serial Configuration Device (also shown in Figure 1). This configuration device is split into two sections: configuration memory, and general purpose memory. The EPCS Controller provides a boot-loader feature that allows the Nios II system to store the main program code in an external EPCS configuration device [1]. In order to configure the Nios in this manner, you must set the reset address to the base address of the EPCS Controller [2]. After reset, the CPU first executes code from the boot-loader ROM, which copies data from the EPCS general-purpose memory region into RAM. Then, the program-control transfers to RAM. The FPGA configuration data is also stored in this configuration memory location [1]. The rest of the memory in this chip is available as general purpose memory. We are using the 16 MB version of this chip, and will therefore have plenty of flash memory to work with (we were originally unaware of how much video-sprite information we would have to store). It is divided into 32 sectors of 512 Kb each, and thus ranges in address from 000000h to 1FFFFFh [3]. The actual Controller Core reserves addresses 000h-0FFh (using 32-bit words) for the boot loader code [1]. The next 7 words are control and data registers (each 32 bits long) [1]. In addition to the Boot-loader ROM and 7 control/data registers on the EPCS Controller Core, there are also 32 general purpose registers [4]. A few of these registers include the stack pointer, frame pointer, global pointer, etc. Any variables within our C code will be stored in the remaining on-chip memory. However, the majority of the data that we store will be the two video buffers. As mentioned previously, we are double buffering, and therefore must store an 8-bit color code for every pixel on the screen in each buffer. Thus, we have two external 512 Kb x 8 SRAM chips to hold this information in addresses 00000h-A1E7Fh respectively. Our MIDI files will be stored on the serial flash device in the general purpose memory, along with any text/video sprites that we would like to store and simply load into our video buffers at the appropriate time. The text sprites that we will need quick access to (such as numbers for the score and the word “Perfect”) will be stored within the VHDL code as an array of std_logic_vectors (the Cyclone II FPGA has 18,752 logic elements available).

Figure 1 [1]

Since we are using an FPGA, there are not port names with a specified number of pins associated with each port, as there would be with a microprocessor. Therefore in the following section I will outline the range of pin numbers corresponding to which side of the FPGA we are interfacing with each external device (there are power and ground pins placed intermittently throughout these ranges, however). Our two SRAM chips take up two complete sides of the FPGA’s I/O pins ranging in values from 180-240, and 120-180, respectively. Another complete side of the chip is consumed by our Digital to Analog Converter (DAC). This interface is using pins ranging anywhere from 60-120. The last side of the chip houses the MIDI input, a MIDI synthesizer chip, the Serial Configuration Device, and our 80 MHz oscillator for our system clock. This side contains pins ranging from 0-60.

Since our VHDL code is synthesized in hardware, it will all be running concurrently. However, our C code is intended to be coded in the event/interrupt driven style. This is due to the fact that once the user selects a song from the menu the processor will parse the whole MIDI file, and then sleep until it either receives a key-press event, or until it is time to send out note timing information to the FPGA. It will loop continuously, until the song is over.

In order to assist the debugging process, we have headers and test points scattered throughout our PC board. All powers and grounds are brought to a header, both in order to test their values, as well as if we will for some reason need to fly-wire a circuit, post-fabrication. Also, both data and address buses as well as the read/write flags of the SRAM chips are brought out to headers so that we can see if they are the correct values that we are setting them to be. We even have about 20 I/O pins being brought out to headers in case we need to test a signal within our code, or even need to add circuitry. Since our hardware design was so complex it became evident that we need as many headers as we could fit, so that we could fully understand our circuit. From a programming perspective, the nice aspect of working with VHDL is that you can simulate your design and view the waveform for each signal. This makes debugging easier, because we will be able to see every signal that we are sending to each external device at a given instant in time.

- Software Design Narrative

Control Unit (VHDL)

This is the highest level module of our code. It is basically a state machine implemented in VHDL that tells the C program when to start looking for MIDI input and which MIDI file it needs to parse at the appropriate time, as well as what should be stored in SRAM at a given time and thus outputted to the screen. The start state corresponds to the start screen that appears on boot-up (simply stating “Press Any Key to Continue”). Once a key is pressed, the state machine moves into the song selection state, which is where the user picks a song to play. Once a song is chosen, the state machine moves into the game-play state, which it will remain in for the duration of the song. This is where the bars (corresponding to a given key) scroll down the screen and the user actually plays the song. Once this stage is completed, the final state is the end of song state, which basically shows the user’s score. This module has been completely written but not yet tested.

Parse MIDI (C)

This module will be coded in C, and accepts the song to be played from the Control Unit. It then accesses the corresponding location in memory and reads each byte until the end of the MIDI file. Based on the note timing information provided in the MIDI file, the program will build a series of vectors that it will then pass to the FPGA at the appropriate moment in time. Each vector sent will be 48 bits long (one bit corresponding to each key) and will simply be a mapping of each key that is pressed at that moment in time during the song. The nice feature of MIDI format is that it simply lists the key-events and at what time offset they occur at. Therefore, in between key-events the program will sleep. This module has not been coded yet, but we have outlined it in pseudo-code.

Watch MIDI (C)

This module will also be coded in C, and is an interrupt that simply watches the MIDI input line. When a key is pressed, the MIDI signal is decoded, and a vector corresponding to the keyboard (one bit for each key) is updated and sent to the FPGA so that it can check the accuracy and show which key was pressed on the screen. This module has not yet been started, because we have not yet looked into how to decode the MIDI signal. This will begin this weekend.

Output Logic (VHDL)

The Output Logic module will be coded in VHDL, and basically determines what information gets put into the SRAM buffers. It can be thought of as a big multiplexer, since it chooses who’s data to use based on which state it’s in as well as what the key-press vectors from the MIDI blocks look like. It is these vectors that will determine which bars fall on the screen (their propagation down the screen will be described below). This block has not yet been programmed, but we have pseudo-code for it.

Audio Out (C)

The Audio Out module simply feeds appropriate MIDI information to the MIDI synthesizer on our PC board if the user pressed the correct key in the given tolerance of time. This module should be fairly simple, as the MIDI chip should do most of the work. It will simply need to take an input vector from the FPGA corresponding to the keys pressed and translate it into something that the MIDI chip can understand. This module has also not yet been programmed.

SRAM1 and SRAM2 (VHDL)

These two blocks of code control both the reading of and writing to SRAM. As mentioned previously, we are double buffering, so at all times we must be reading from one SRAM, and using the other to write updates to. Reading the color codes from the SRAM and sending them to the VGA Out block will be the easy part (just a matter of timing signals correctly). However, in order to correctly update the falling blocks, we must keep a two dimensional array, mapping out the whole screen at any given instant of time. This array will be 50 x 48 (one column per key). After a given amount of time has passed (based on the tempo of the song), each row will propagate down a single row, and this mapping will be reflected in the SRAM that we are currently writing to. Fortunately, since our game does not involve a large amount of movement at any given point in time, we do not have to flip between SRAM chips the full 60 times that the monitor refreshes every second. Instead, we can read from one SRAM chip several times in a row, thus giving us plenty of time to write to the second chip. Therefore, we would only flip the page (and thus refresh the display) around 10 times per second, which should be sufficient for our purposes (if not, we will increase the number of refreshes). We have begun coding this module, but it is not yet complete.

Scoring Logic (VHDL)

The Scoring Logic module simply checks a certain tolerance of pixels that we will allow the key to be pressed in relation to the falling bar (for instance the user should press the key at the bottom of the screen + or – 10 pixels). If the bar is in that pixel region when the key is pressed, their score will be increased, otherwise it will be reduced. This is a simple module that we have pseudo-code for, and will be coded once the bars are able to fall down the screen.

VGA Out (VHDL)

The VGA Out module runs continuously the entire time that our device is turned on. It is configured for 800 x 600 resolution, and makes sure that the correct RGB values are output for each pixel every 25 ns (pixel clock for that resolution) [5]. It is also responsible for sending the appropriate blanking signal, and vertical and horizontal sync pulses at the specified times for 800 x 600 resolution during each refresh [5]. This code uses a vertical counter and a horizontal counter to track its position on the monitor and grabs a new color code from SRAM every 25 ns, changing the RGB values based on the lookup table corresponding built into our VHDL code.  This section of the code is completely finished and tested.

- Summary

This Software Design Narrative has shown in detail how our project restrictions have guided our coding as a whole for our project (Hooked on Harmonix). The main software will be coded in the event/interrupt style of coding, with the processor sleeping until it needs to send out updates to the FPGA. The VHDL code, on the other hand, will be running concurrently and handling all video output considerations. A detailed mapping of the various locations in memory has also been outlined. This helped increase our knowledge of how our program will be working, but most of the memory storage considerations will be taken care of by the Nios II IDE compiler. The pin mappings for each external component have also been briefly described, as well as how we plan to debug each circuit in post-fabrication of our PCB. Finally, each module of code was described in detail, as well as their completion status. The software design for Hooked on Harmonix has been carefully and thoroughly thought out, which should make the actual coding process much faster. This was essential, because with all of our hardware considerations we were not given as much time for software as expected.

List of References

- Altera Corporation, “EPCS Device Controller Core”, [Online Document], October 2007, http://www.altera.com/literature/hb/nios2/n2cpu_nii51012.pdf

- Altera Corporation, “Instantiating the Nios II Processor in SOPC Builder”, [Online Document], October 2007, http://www.altera.com/literature/hb/nios2/n2cpu_nii51004.pdf

- Altera Corporation, “Configuration Handbook Volume 1, Section 1, Chapter 4: Serial Configuration Devices Data Sheet”, [Online Document], August 2007, http://www.altera.com/literature/hb/cfg/cyc_c51014.pdf

- Altera Corporation, “Introduction to the Altera Nios II Soft Processor”, [Online Document], http://users.ece.gatech.edu/~hamblen/DE1/DE1_CDROM/DE1_tutorials/tut_nios2_introduction.pdf

- Nathan Ickes, “VGA Video Output”, [Online Document], http://www-mtl.mit.edu/Courses/6.111/labkit/vga.shtml

Appendix A: Flowchart/Pseudo-code for Main Program

Load program/sprites from flash

Song chosen?

Load/parse song

Show Final Score

Display start screen and song menu

Song over?

Pass notes to FPGA and watch MIDI input

No

No

Yes

Yes

Appendix B:  Hierarchical Block Diagram of Code Organization

VGA out

(VHDL)

SRAM1

(VHDL)

Ctrl Unit

(VHDL)

Parse MIDI

(C)

Watch MIDI

(C)

Output Logic

(VHDL)

SRAM2

(VHDL)

Audio Out

(C)

Scoring Logic

(VHDL)