Homework 12:  Ethical and Environmental Impact Analysis

Due: Friday, November 16, at NOON

Team Code Name: Hooked on Harmonix                                                               Group No.  2

Team Member Completing This Homework: Tom Bottonari

e-mail Address of Team Member: tbottona @ purdue.edu

NOTE:  This is the last in a series of four “professional component” homework assignments, each of which is to be completed by one team member.  The completed homework will count for 20% of the individual component of the team member’s grade.  The body of the report should be 3-5 pages, not including this cover page, references, attachments or appendices.

Evaluation:

| ## SCORE | ### DESCRIPTION |
| --- | --- |
| 10 | # Excellent – among the best papers submitted for this assignment. Very few corrections needed for version submitted in Final Report. |
| 9 | # Very good – all requirements aptly met. Minor additions/corrections needed for version submitted in Final Report. |
| 8 | # Good – all requirements considered and addressed. Several noteworthy additions/corrections needed for version submitted in Final Report. |
| 7 | # Average – all requirements basically met, but some revisions in content should be made for the version submitted in the Final Report. |
| 6 | # Marginal – all requirements met at a nominal level. Significant revisions in content should be made for the version submitted in the Final Report. |
| * | # Below the passing threshold – major revisions required to meet report requirements at a nominal level. Revise and resubmit. |

* Resubmissions are due within one week of the date of return, and will be awarded a score of “6” provided all report requirements have been met at a nominal level.

Comments:

Comment from the grader will be inserted here.

- Introduction

Hooked on Harmonix combines a learning experience for the art of playing the piano with the entertaining environment of a video game into one package. Due to the docile nature of this gaming system, however, it poses very few ethical/environmental dangers over the course of its life-cycle. In fact, since this device is inanimate and produces no harmful chemicals/gases, the only environmental concerns associated with it occur pre/post life-cycle. This environmental impact is virtually the same that is common to most products using electrical components and a printed circuit board. For example, the use of substances such as lead and mercury in manufacturing can be detrimental to the environment if not disposed of properly, and since this device is available to the average person, it will most likely be thrown away in the garbage when it becomes no longer functional. Therefore, steps must be taken during the manufacturing stages of design to protect against such situations. Moreover, from an ethical standpoint, since the device has little possibility of injuring the user and no security issues, the only ethical issues that arise are that of copyright violations for the music that is displayed, as well as the fact that we are interfacing to devices that the user owns, thus creating a potential for damaging them. To ignore the rights of the musician to protect his/her work as an artist or to fail to protect against damaging devices that interface with this product would be a clear violation of ethics.

- Ethical Impact Analysis

As previously mentioned, the Hooked on Harmonix gaming console does not pose any immediate risk of injury or security to the average user, but as is with almost all electrical devices there are certain situations that must be taken into account as a safety precaution. For example, steps must be taken in order to properly handle the case of a power surge or possibly a short across power and ground that causes a sudden spike in current. This could not only severely heat or even damage the device itself, but from the ethical standpoint this current spike could feedback into the external devices that are interfaced with it such as the monitor, speakers, and keyboard. Therefore, if not protected against, a failure in the Hooked on Harmonix device could potentially cause damage to other expensive equipment belonging to the end user. As bad as this may seem, however, these issues can be easily protected against. For example a metal oxide varistor (MOV) could be placed across the power input that clamps at a specified voltage and effectively behaves as a surge protector. Similarly, in order to protect against shorts within the gaming device, varistors could be placed on the outputs to the monitor and speakers that would prevent the analog signal from spiking higher than a certain threshold, thus containing the short within the product. As for the keyboard, the only point of connections is through the MIDI input line, which is optically isolated from the circuit within the device, so if the isolator were to fail from a spike in voltage the keyboard would be disconnected.

Another issue that could potentially damage the aforementioned interfaced monitor is if some malfunction in the FPGA (Field Programmable Gate Array) caused the horizontal or vertical syncing signals to exceed their respective maximum frequency tolerances (1). Corruption of the clock frequency, or perhaps even a short of the power to the FPGA could cause this. However, a simple solution to the problem would be to pass the horizontal sync signal through a band-pass filter that allows for frequencies within a given tolerance, and to pass the vertical sync signal through a low pass filter set to the specified maximum frequency allowed. Therefore, in the event of too high of a frequency, the signals would be driven low and no damage would be done to the monitor.

No ethical analysis would be complete without the analysis of the target user. Since Hooked on Harmonix is intended to be used in the home, testing in extreme temperature conditions is perhaps overkill. However, due to the fact that it will most likely be within reach of small children or pets, a certain amount of childproofing of the enclosure is required in order to be ethically sound. This will prevent such things as a curious child putting his/her fingers through any holes or lifting the lid off and poking around inside, and even help protect against spilled liquids. There will still have to be holes for the ports to stick out of, but as long as the enclosure is fairly tight against the ports and the lid is securely fastened down, these issues should be avoided. Although this is not a high voltage device, a good idea would be to place a warning label on the lid describing the dangers of tampering with electrical circuitry, incase one were to open it up. This issue could even be reinforced in the user's manual.

Aside from design issues, the very fact that this device is showing you how to play music written by another individual, as well as the fact that it is potentially using said music to turn a profit, constitutes the need for permission from the artist. Using their work without their permission is not only a clear violation of ethics, but it is against the law. Therefore, permission will have to be obtained for all copyrighted songs used in Hooked on Harmonix.

Finally, since the product is inanimate and requires little physical contact with the user, any logic bugs in software/hardware code would not pose any threat to the user. As mentioned previously, if the syncing signals were programmed out of tolerance the monitor could incur some damage, but this is a pretty obvious bug to find and would more likely occur due to some corruption within the FPGA. All other software bugs would simply cause the game to misbehave, which would most likely end in losing the customer due to dissatisfaction.

- Environmental Impact Analysis

Since this product will spend most of its life indoors and produces no environmental dangers as a byproduct, all environmental issues will have to be taken care of during manufacturing and disposal. Two main issues arise in the production and disposal of such an electrical device. These issues are basically the overuse of natural substances such as copper that are in limited supply, and the use of certain substances that are detrimental to society. Since someone uneducated in the composition of electrical components will most likely be using this product, they will most likely throw it in the trash at the end of its life-cycle, thus exposing those detrimental materials to the environment. Therefore, the second of the two issues is especially important.

The issue of recycling materials used in the design of this product can be addressed both in the manufacturing as well as the disposal stages of its life-cycle. Since other companies will manufacture most of the components in the design as well as the printed circuit board, it is important to choose carefully which companies to work with. Metallic elements on a printed circuit board such as copper, nickel, tin, lead, gold, palladium, and silver can all be recovered by recycling (2), and when millions of these boards are ordered from a company any little bit to conserve the earth's natural resources helps. Since the end user has control over how the device is disposed of, it is important to do as much as possible in the manufacturing stage. However, even though the user has control over how the device is disposed of, various steps can be made to make sure that they are informed. For instance, instructions can be given in the user's manual on the proper method of disposal so that as much of the product gets recycled as possible. Furthermore, since not everybody reads the user's manual, a warning label would most likely be put on the device stating that the components inside are possibly detrimental to the environment and should be disposed of properly, referencing the user's manual as to the proper methodology.

Unfortunately, one cannot always rely on others to make good decisions regarding the environment, and therefore steps must be taken in order to anticipate improper disposal. This is done in the manufacturing stage of the design by things such as choosing ROHS compliant parts. The ROHS directive restricts the use of lead, mercury, cadmium, hexavalent chromium, polybrominated biphenyls, and pollybrominated diphenyl ether to .1% (or .01% for cadmium) by weight of homogeneous material (3).  As an indication of the importance of such compliance, lead poisoning can cause anemia, damage to your nervous system or brain, kidney disease, and impotence (4), and although we do not always see the effects of this, the animals whose habitats are filled with our lead waste most certainly do.

- Summary

Although many companies these days do not take up the initiative to help save the environment, it is the responsibility of those manufacturing the products that we use in everyday life to both conserve natural resources as well as limit the amount of harmful substances that are eventually dumped in our ecosystem. Many times when people think of ethical responsibilities they think about the unfair treatment of humans, but in actuality these two issues overlap a great deal. Just as there is an inherent responsibility for companies to protect their customers, it is also a question of ethics that guides the environmental considerations in the design of a product. Hooked on Harmonix is a gaming system whose functionality has no negative effects on the environment, but much still needs to be done to ensure safety in society and nature (as outlined in this document) should this product be taken to production. Steps such as protective circuitry and childproof enclosures would need to be taken to protect the user, while artist consent would need to be obtained in order to display information regarding songs to be displayed. Furthermore, an emphasis on environmental integrity is imperative for the furtherance of society, especially with the technological direction in which we are headed. Without some methods of conservation, we will soon run out of the materials necessary for the composition of such devices. This is not always the cheapest route, but it far outweighs the negative press associated with violating environmental regulations. However, as much as these decisions are business driven, the responsibility still lies within the company to make good decisions from an ethical and environmental standpoint.

List of References

- Tomi Engdahl (Tomi.Engdahl@iki.fi), "VGA to Workstation Monitor FAQ", 2000, http://www.epanorama.net/documents/vga2rgb/basics.html

- Martin Tarr, "PCB recycling issues",  [Online Document] http://www.ami.ac.uk/courses/topics/0113_prei/index.html

- Wikipedia, "Restriction of Hazardous Substances Directive", http://en.wikipedia.org/wiki/Restriction_of_Hazardous_Substances_Directive

- Thomas Detwyler (tdetwyle@uwsp.edu), "Lead in the Environment", October 2000, http://www.uwsp.edu/geo/courses/geog100/Lead-InEnv.htm

IMPORTANT:  Spend a significant amount of time on this – see articles provided on the course web site.  Use standard IEEE format for references, and CITE ALL REFERENCES listed in the body of your report.  Any URLs cited should be “hot” links.