# Reliability and Safety Analysis

# Year: 2015 	Semester: Spring	Team: 13 	Project: CoDDeLo

# Creation Date: 04/03/2015				Last Modified: October 7, 2015

# Author: <redacted>					Email: rdacted@purdue.edu

# Assignment Evaluation:

| Item | Score (0-5) | Weight | Points | Notes |
| --- | --- | --- | --- | --- |
| Assignment-Specific Items |  |  |  |  |
| Reliability Analysis |  |  |  |  |
| MTTF Tables |  |  |  |  |
| FMECA Analysis |  |  |  |  |
| Schematic of Functional Blocks (Appendix A) |  |  |  |  |
| FMECA Worksheet (Appendix B) |  |  |  |  |
| Writing-Specific Items |  |  |  |  |
| Spelling and Grammar |  |  |  |  |
| Formatting and Citations |  |  |  |  |
| Figures and Graphs |  |  |  |  |
| Technical Writing Style |  |  |  |  |
| Total Score |  |  |  |  |

# 5: Excellent 	4: Good     3: Acceptable    2: Poor     1: Very Poor    0: Not attempted

# Comments:

# Reliability Analysis

# dsPIC33FJ256GP710A

# The dsPIC33FJ256GP710A [1] is used to control the entire system. It was chosen for reliability analysis because of its complexity. The failure per 10^6 hours (λP) and mean time to failure (MTTF) was calculated based on the formulas outlined in MIL-HDBK-217F [2].  I used the model of digital CMOS IC microprocessor model for this microcontroller. λP = (C1 · πt + C2· πe) · πl · πq. The MTTF and λP is in the expected range. A failure of this microcontroller would not directly cause harm to the users.

Table 1, microcontroller analysis.

| # Parameter name | # Description | # Value | # Comments |
| --- | --- | --- | --- |
| # C1 | # Die complexity | # 0.14 | # PIC controller |
| # πt | # Temperature coeff. | # 1.5 | # PIC controller |
| # C2 | # Pin/Package Constant | # 0.052 | # Nonhermetic SMT with 100 pins |
| # πe | # Environmental Constant | # 0.5 | # GB |
| # πl | # Learning Factor | # 1 | # The IC is more than two years in production. |
| # πq | # Quality Factor | # 10 | # Commercial components |
| # Entire design: |  |  |  |
| # λP |  | # 2.36 |  |
| # MTTF |  | # 0.424 |  |

MCP1700

The MCP1700 [3] voltage regulator is used to provide power to the digital circuits. If this voltage regulator fails to operate normally, there will be insufficient or excessive amount of power deliver to other digital components in our system, which would cause mistakes in logic computations. Incorrect logic computation would make some component, such as keypad, LCD, Xbee and motor, operate in an unexpected way, which would damage the entire system and users.

The failure per 10^6 hours is calculated by λP = (C1 · πt + C2· πe) · πl · πq. The model of CMOS Switch-Mode Regulator is used. The MTTF and λP is in the expected range. A failure of this microcontroller would not directly cause harm to the users.

Table 2, voltage regulator analysis.

| # Parameter name | # Description | # Value | # Comments |
| --- | --- | --- | --- |
| # C1 | # Die complexity | # 0.01 | # 1 to 100 bipolar transistors is used |
| # πt | # Temperature coeff. | # 0.1 | # CMOS, Tj= 25 C |
| # C2 | # Pin/Package Constant | # 0.0012 | # 3 pin SMT |
| # πe | # Environmental Constant | # 0.5 | # GB |
| # πl | # Learning Factor | # 1 | # The IC is more than two years in production. |
| # πq | # Quality Factor | # 10 | # Commercial components |
| # Entire design: |  |  |  |
| # λP |  | # 0.016 |  |
| # MTTF |  | # 62.5 |  |

Xbee Module

The Xbee S1 wifi module contains MC 13213 HS08 microcontroller which dominate the complexity of this module. Also, the Xbee module is a key component to let our system to communicate with a database and quadcopters.  Given this, it is important to analyze the reliability of the Xbee. The following data is obtained from the MIL_HDBK_217F [2] and the MC13213 datasheet[4]. The failure per 10^6 hours is calculated by λP = (C1 · πt + C2· πe) · πl · πq. The model of digital CMOS IC microprocessor is used. The MTTF and λP is in the expected range. A failure of this microcontroller would not directly cause harm to the users.

# Table 3, Xbee module analysis.

| # Parameter name | # Description | # Value | # Comments |
| --- | --- | --- | --- |
| # C1 | # Die complexity | # 0.14 | # 8-bit microcontroller |
| # πt | # Temperature coeff. | # 0.98 | # maximum operating temperature is 85° C |
| # C2 | # Pin/Package Constant | # 0.036 | # Nonhermetic SMT with 71 pins |
| # πe | # Environmental Constant | # 0.5 | # GB |
| # πl | # Learning Factor | # 1 | # The IC is more than two years in production. |
| # πq | # Quality Factor | # 10 | # Commercial components |
| # Entire design: |  |  |  |
| # λP |  | # 1.552 |  |
| # MTTF |  | # 0.64 |  |

# SUMMARY

# The MTTF of all three components all more than half million hours which are long enough for our machine. The possible methods to improve the reliability of our design is to use microcontroller and other ICs as simple as possible. We can also design a cooling system to reduce the temperature coefficient.

# Failure Mode, Effects, and Criticality Analysis (FMECA)

# There were three criticality levels for this project, low medium and high. High refers to failures which cause injury to users in a catastrophic way, such as burning and toxic gas releasing. Medium refers to failures which has a small probability to cause injury to users by mechanical collision. Low refers to negligible failure which only cause the system to shut down and does not do any damage to users. The acceptable failure rate for high level failure is 10-10. For medium and low level failure, the failure rate is 10-6. I assume the users read the user manual thoroughly before using our machine. We also assume the electrical system of our design will not be exposed in raining, snowing or other catastrophic weather.

# 3.0 Sources Cited:

# dsPIC33FJXXXGPX06A/X08A/X10A Data Sheet, Microchip Technology Inc., 2011.

# “Military Handbook Reliability Prediction of Electronic Equipment” Department of     Defense. Washington DC. MIL-HDBK-217F, Dec. 2, 1991.

# MCP1700 Low Quiescent Current LDO, Microchip Technology Inc., 2013

# MC13211/212/213 ZigBee™- Compliant Platform - 2.4 GHz Low Power Transceiver for the IEEE® 802.15.4 Standard plus Microcontroller, Rev 1.8, Freescale Semiconductor, 20

# Appendix A:  Schematic Functional Blocks

Figure 1, power supply circuit.

Figure 2, keypad circuit.

Figure 3, in-circuit programmer circuit.

Figure 4, reset circuit.

Figure 5, logic level converter circuit.

Figure 6, Xbee module circuit.

Figure 7, crystal oscillator circuit.

Figure 8, shift register circuit.

Figure 9, microcontroller circuit.

# Appendix B:  FMECA Worksheet

# Table 4, power supply circuit FEMCA.

| Failure No. | Failure Mode | Possible Causes | Failure Effects | Method of Detection | Criticality | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Voltage drop | C8, C28, C1, C2, C3, C4 | Errors appear in logic computation. Some components may turns off. | Observation, Display and transmitting errors on LCD and Xbee. | Low |  |
| 2 | Voltage regulator fails short | 2 voltage regulators | Damage all ICs and overheat the system. | Observation, smoke/burning, excessive heat | High | The overheat on the electrical components would cause burning and toxic gas releasing from PCB |

# Table 5, keypad circuit FEMCA.

| Failure No. | Failure Mode | Possible Causes | Failure Effects | Method of Detection | Criticality | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | No input received from keypad | EDE 1144 encoder (IC2), Crystal oscillator | User cannot enter their username and password | Observation, Measure the output voltage from the keypad encoder | Low | Assuming the resistors are not open-circuited and supply voltage is normal. |

# Table 6, in-circuit programmer circuit FEMCA.

| Failure No. | Failure Mode | Possible Causes | Failure Effects | Method of Detection | Criticality | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | No voltage supply to in-circuit programmer | C10 | Cannot program the microcontroller | Observation, errors appear when program the microcontroller | Low | Assuming supply voltage is normal. |
| 5 | No data received from in-circuit programmer | Damaged programming cable, damaged RJ11 connector | Cannot program the microcontroller | Observation, errors appear when program the microcontroller | Low | Assuming supply voltage is normal. |
| 6 | Large noise appears on the output of the in-circuit programmer | Opened bypass capacitor | Unpredictable | Observation, wrong software program might be performed | Low |  |

# Table 7, reset circuit FEMCA.

| Failure No. | Failure Mode | Possible Causes | Failure Effects | Method of Detection | Criticality | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 7 | Reset signal is always high | Damaged pushbutton | Cannot reset the system | Observation | Low |  |
| 8 | Reset signal is always low | Shorted bypass capacitor C9 | The system is always resetting. The system cannot function. | Observation | Low | Assuming pushbutton is in good status. |

# Table 8, logic level converter circuit FEMCA.

| Failure No. | Failure Mode | Possible Causes | Failure Effects | Method of Detection | Criticality | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 9 | Output cannot reach 5V | Damaged logic converter IC | Cannot control the component that connect to the logic converters (LCD and motor drivers). | Observation, Measure the output voltage from logic converter. | Medium | Assuming supply voltage is normal. |
| 10 | Large noise appears on the output of the logic converter | Opened bypass capacitor | Unpredictable | Malfunction may appears on some components | Medium |  |

# Table 9, Xbee module circuit FEMCA.

| Failure No. | Failure Mode | Possible Causes | Failure Effects | Method of Detection | Criticality | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 11 | Failure to send data | IC on Xbee module, | No communication between our system and a database | Observation | Low | Assuming supply voltage is normal. |

# Table 10, crystal oscillator circuit FEMCA.

| Failure No. | Failure Mode | Possible Causes | Failure Effects | Method of Detection | Criticality | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 12 | No signal received from OSC1 & OSC2 of the microcontroller | Damaged crystal (Q1) | Real time application will not accurate | Observation | Low |  |

# Table 11, shift register circuit FEMCA.

| Failure No. | Failure Mode | Possible Causes | Failure Effects | Method of Detection | Criticality | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 13 | Wrong parallel data output from the shift register | Microcontroller SPI data output and clock, Damaged shift register IC | Errors appear on LCD | Observation | Low |  |
| 14 | No parallel data output from the shift register | Damaged shift register IC | No display on LCD | Observation | Low | Assuming supply voltage is normal. |

# Table 12, microcontroller circuit FEMCA.

| Failure No. | Failure Mode | Possible Causes | Failure Effects | Method of Detection | Criticality | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 15 | No signal come out of the microcontroller | Damaged capacitor on VDD/VCORE pin | Unpredictable | Every component does not function. | Low | Assuming supply voltage is normal. |
| 16 | No supply voltage goes into VCC pin | Shorted bypass capacitors | Unpredictable | Every component does not function | Low | Assuming supply voltage is normal. |
| 17 | Large noise appears on the output of the microcontroller | Opened bypass capacitor | Unpredictable | Malfunction may appears on some components | Medium |  |