- Notes:

- [name] indicates ‘name’ group is appropriate for the given task

- Aim to have tasks listed complete by the end of the given week

- Week 1 - Finalize Ideas

- Nail down your final details as a group

  - What sensors will you use?

  - What peripherals will you need?

  - You can always assume more than necessary, but don’t assume less

  - What power voltages will you use? 12V, 5V, 3.3V, higher, lower?

  - What will supply power?  Battery, A/C, wall wart?

  - Will you use a CPU?

- Week 2 - Select major components

- Pick a family of microcontrollers (e.g. - dsPIC, PIC32, PIC24, etc)

- Get a microcontroller dev board and begin testing peripherals [embedded software]

  - Blink an LED (GPIO)

  - Control blink speed with a potentiometer (AtD + Timer)

  - Debug via RS232 (RS232/UART)

  - Also implement other interfaces as appropriate (e.g. - I2C, external interrupts)

- Select power components [hardware]

  - There are very few reasons not to use a switching regulator

- Order some parts to prototype [group]

- Week 3 - Begin prototyping

- Finalize and order any parts you haven’t yet [group]

- If you will use a CPU (Atom, Beagle Board, etc.), it is appropriate to order it here [group]

- Get some prototyping boards, and begin prototyping microcontroller interfacing to sensors, etc. [embedded software]

  - Work on the hard things or things with weird or complex schematics first (e.g. - battery charger IC, GPS, etc.)

  - Do AtD last because it’s easy

- Begin prototyping power circuitry [hardware]

- Finalize major components and begin selecting non-major components

  - Schottky diodes, tantalum capacitors, etc.

  - Don’t forget that you will need a MAX3232 or equivalent to communicate between a PC and a micro via RS232!!!

- Week 4 - Continue prototyping

- Power circuitry prototyping should be done or nearly done [hardware]

- You should have something to show with sensor interfacing [embedded software]

- Week 5 - Paper schematics

- Start developing schematics (at least on paper) for all prototypes and planned prototypes [whomever is best suited]

  - This should include part values, etc.

  - Be sure to make notes here about any special cases put in part datasheets

- Design footprints for parts library for PCB [PCB team]

- Test CPU if appropriate [CPU team, or as appropriate]

- Week 6 - Digital schematics

- Put schematics into electronic form [PCB team]

  - Should be nearly finished with entire schematic.

- Put major (and minor, if possible) components on a PCB to check sizing [PCB]

- Begin writing test software for CPU if appropriate [CPU software team]

- Week 7 - PCB layout

- Prototyping should be near completion or complete [embedded software]

- Begin routing some traces on the PCB [PCB]

- Week 8 - Design Reviews!

- Prototyping should be fully complete [embedded software]

- PCB should be done and ready for revision