Homework No. 7 and Problem Set for Lab

Due Date: Wednesday, March 1, 2017

Problem No. 1

A C8x18.75 member is to be used as a tension member. The channel is bolted to a 3/8 in. thick gusset plate with 5/8 in. diameter A325 bolts. The tension member is A572 Grade 50 steel, and the gusset plate is A36 steel. Assume that threads are included in the shear plane.

a. Compute the design strength of the connection assuming bolt shear failure governs

b. Compute the design strength assuming bearing failure in the channel member governs

c. Compute the design strength assuming bearing failure in the gusset plate governs

d. Are all the edge distance and spacing requirements satisfied? What is the governing design strength of the connection

Problem No. 2

Problem No. 3

A C9 x 20 is used as a tension member and connected to a 1/2 in. thick gusset plate. A572 Gr. 50 is used for the tension member, and A36 steel for the gusset plate. The member is designed to carry 40 kips of nominal dead load and 80 kips of nominal live load.

(a) Design the connection using Group A bolts.

(b) Detail the connection bolt layout and provide a sketch

(c) Compute the design strength of the member with your designed connection. Consider all possible limit states. Determine if you need to redesign your connection, but leave as is.

Problem No. 4

Design a splice for the W8x28 member (made of A992 steel) to carry a factored load of 300 kips. Consider all possible limit states, and provide a detailed drawing of the splice connection.