# Appendix A: Aerodynamics

### A.1 Speeds used in Calculations

Stall Speed: 20 ft/s

Cruise Speed: 25 ft/s

Max Speed: 30 ft/s

### A.2 Reynolds Number Calculations

Re=ρUcμ

Where ρ is the density, U is the velocity, c is the chord, and μ is the kinematic viscosity.

Reynolds numbers calculated ranged from 186279-279419

# A.3 Warner’s method of determining three-dimensional Coefficients

dCldαb=11dCldα+nπAR+β2Equation A.1

Lift Curve Slope:

Where dCldα is the lift curve slope and AR is the aspect ratio

## A.4 Zero Angle Lift Coefficient

Equation A.2

Zero Lift Coefficient:

CLo=CLowf+CLαhηhShS(ih−εoh)Where CLo is the zero angle lift coefficient, CLowf is the zero angle lift coefficient of the wing fuselage, CLoh is the zero angle lift coefficient of the horizontal tail, ηh is the, Sh is the area of the horizontal tail, S is the area of the wing, ih is the incidence of the horizontal tail, and εoh is the downwash angle due to the horizontal tail.

Equation A.3

CL=CLα(α−αol)+CLo+CLδeδeThree Dimensional Coefficient of Lift

Where Clα is the lift curve slope, alpha is the angle of attack, αol if the zero lift angle of attack, CLo is the previously calculated zero lift coefficient of lift, Clδe is the coefficient of lift due to elevator deflection, and delta e is the deflection of the elevator.

## A.5 Zero Angle Coefficient of Drag

Equation A.4

Zero Angle Coefficient of Drag

CDo=Cdwing+Cdfuselage+Cdvtail+Cdhtail+Cdlg+Cdstrut

Where Cdwing is the coefficient of drag of the wing, Cdfuselage is the coefficient of drag for the fuselage, Cdvtail is the coefficient of drag due to the vertical tail, Cdhtail is the coefficient of drag due to horizontal tail, Cdlg is the coefficient of drag due to the landing gear, Cdstrut is the coefficient of drag due to the struts.

## A.6 Induced Drag

Equation A.5

Induced Drag

CDi=CL2eπAR

Where CL is the coefficient of lift, e is Oswald’s efficiency factor, and AR is the aspect ratio.

## A.7 Oswald’s efficiency factor for biplanes

Equation A.6

Oswald’s efficiency factor

e=μ2(1+r2)μ2+2σμr+r2

Where μ is kinematic viscosity

## A.8 Three-Dimensional Coefficient of Drag

Equation A.7

Coefficient of Drag

CD=CDo+CDi

Where CD is the total coefficient of drag, Cdo is the zero lift coefficient of drag, and Cdi is the induced drag.

## A.9 Roskam’s Method of determining three-dimensional coefficients

Equation A.8

Coefficient of Lift

CL=CLo+CLαα

Equation A.9

Lift Curve Slope

CLα=CLαwf+CLαhηηShtS1−dεdα

Equation A.10

Lift Curve Slope-wing fuselage

CLαwf=KwfCLαw

Equation A.11

Downwash on the wing due to the horizontal tail

dεdα=4.44KAKλKh(cosΛc4)1.19Where  dεdαis the downwash on the wing due to the horizontal tail Λc4 is the quarter chord sweep of the horizontal tail, CLαw is the lift curve slope of the wing, and CLαwf   is the lift curve slope of the wing-fuselage combination.

Equation A.12

Form Factor Utilized in downwash equation

KA=1AR−1(1+AR1.7)

Equation A.13

Form Factor Utilized in downwash equation

Kλ=1710−3λ

Equation A.14

Form Factor Utilized in downwash equation

Kh=1−hhb2lhb(13)

CLo=(0.5(iwu+iwl)−αoLw)CLαwfEquation A.15

Lift Coefficient Zero Angle of Attack

## A.10 Three-Dimensional Lift Curve Slope

Equation A.16

Lift Curve Slope

CLα=Clα1+57.3ClαπARe

## A.11 Three-Dimensional Coefficient of Drag

Equation A.17

Induced Drag

CDi=CL2eπAR

Equation A.18

Oswald’s Efficiency Factor

e=μ2(1+r2)μ2+2σμr+r2

Equation A.19

Drag Coefficient at Zero Angle of Attack

Cdo=(Cdfuselage+Cdwing+Cdhorizontaltail+Cdverticaltail+Cdstrut)1.18

Equation A.20

Coefficient of Drag

## CD=CDo+CDi

## A.12 Endurance Parameter

To solve for the best endurance speed the following equations were used

Equation A.21

Coefficient of Lift-Coefficient of Drag Relationship

CL32CD=number

This number was calculated and placed into the coefficient of drag equation.

Equation A.22

Coefficient of Drag

CD=CDo+kCL2

The coefficient of drag equation was manipulated to take the following form

Equation A.23

Manipulated Coefficient of Drag Equation

CL32CDo+kCL2=number

Equation A.24

Lift Equation

L=W=.5ρSCLV2

This equation was manipulated to solve for velocity

Equation A.25

Lift Equation Manipulated

V2=2WρSCL

Equation A.26

Equation for Best Endurance Velocity

V=2(WS)ρCL

Where V is the velocity, W is the weight, rho is the density, CL is the coefficient of lift, Cdo is the zero angle coefficient of drag, k 1/(piARe), where AR is the aspect ratio, and e is Oswald’s efficiency factor.

## A.13 Coefficient of Moment Calculations

Equation A.27

Zero Angle of Attack Coefficient of Moment

Cmo=Cmw+chShcwSwCmh+xcwCLαw−lhShcwSw(CLoh+CLαh(ih−εo))

Equation A.28

Moment Coefficient due to Angle of Attack

Cmα=xcwCLαw−lhShcwSwCLαh(1−dεdα)

Equation A.29

Coefficient of Moment Equation

Cm=Cmo+Cmαα+Cmδeδe

Where Cm is the coefficient of moment, Cmw is the coefficient of moment of the wing, ch is the chord of the horizontal tail, Sh is the area of the horizontal tail, cw is the chord of the wing, Sw is the area of the wing, Cmh is the coefficient of moment of the horizontal tail, x is the neutral point, Clalphaw is the lift slope curve of wing, lh is the length of the horizontal tail, xacwingbody is the aerodynamic center of the wing body combination, Cmo is the zero lift coefficient of moment, Cmdeltae is the coefficient of moment due to deflection.

## A.14 Locating the Center of Gravity

Equation A.30

Center of gravity location

x=xacwingbody+CLαhCLαwηhShSwxach(1−dεdα)1+CLαhCLαwηhShSw(1−dεdα)xcg=x−SMEquation A.31

Neutral Point location

Where xcg is the location of the center of gravity, x is the location of the neutral point, and SM is the static margin.

A.15 Aerodynamic figures

Plots utilized in Airfoil Selection

Figure A.1: Coefficient of Lift vs. Angle of Attack at RE=102600

Figure A.2: Coefficient of Lift vs. Angle of Attack at RE=203800

Figure A.3: Coefficient of Lift vs. Angle of Attack at RE=301200

Figure A.4: Two Dimensional Drag Polar at RE=102600

Figure A.5: Two Dimensional Drag Polar at RE=203800

Figure A.6: Two Dimensional Drag Polar at RE=301299

The following website was referenced when searching for airfoils: www.nasg.com

Sigma and Beta curve Fits

Figure A.7: Sigma vs. Gap/span curve fit

Figure A.8: Beta vs. Gap/chord curve fit

CMARC and Xfoil limitations and explanation

Two aerodynamics panel codes used by Orion Aerospace were XFoil and CMARC.  A panel code takes a full model and cuts it into pieces, or panels, and solves equations for the airflow over each panel.  Accurate model analysis can be accomplished more easily with a panel code than creating a model for wind tunnel analysis.  XFOIL is a program for the design and analysis of subsonic isolated 2-D airfoils.  CMARC is a panel code for evaluating full 3-D models.

XFoil was used to compare the 2-D aerodynamics of candidate airfoils.  The lift and drag values were calculated by XFoil and compared.  XFoil is inaccurate at low Reynolds numbers.  This is because laminar separation bubbles occur on the airfoil at low Reynolds numbers and XFoil is unable to handle them.  Thus XFoil is not the appropriate tool for analyzing most radio-controlled aircraft because of their low Reynolds number flight regimes.

A CMARC model of SID-5 was also constructed.  CMARC is not an easy program to use and it takes months to get a full aircraft modeled and working properly.  Without prior knowledge of CMARC the task was virtually impossible to accomplish during the course of the semester while designing the aircraft to be modeled and taking several other courses.  The model was made to work at cruise angle of attack and velocity.  Lift and drag were calculated to be the same as the sizing code calculated.  There were several errors still in the file causing inaccurate results at any other angle of attack.

Without taking a course on CMARC before taking senior design it is not wise to attempt a CMARC model.  Many hours were put into the CMARC with few worthwhile results.  The hours would have been better utilized in other ways.

Figure A.9: Coefficient of Pressure calculated in CMARC

Figure A.10: Three-Dimensional L/D vs. angle of attack

B here

### Appendix C: Structures

Structures code:

To find the forces and loads on the aircraft and to be able to get an idea of the affects of these forces, a matlab code was written.  The code first finds the wing loading on the aircraft taking into account a load factor of 3.75. The wing loading was calculated using Schrenk’s approximation.

Schrenk’s approximation takes into account the trapezoidal method (equation C.1 & C.2) and the elliptical method (equation C.3) for wing loading.

EquationC.1:q(y)=q(0)1−2yb1−λEquationC.2:q(0)=w∗3.75bEquationC.3:q(y)=4Sb∗pi1−2yb2The result is shown below in Figure C.1.

Figure C.1:  Wing loading along the wingspan

Once the wing loading was found, the shear forces could be found.  The shear forces were calculated by integrating the wing loading. The result is shown below in Figure C.2.

Figure C.2:  Shear Force along wingspan calculated by matlab code.

Once the shear force along the wing was found, it could be integrated to find the moment felt along by the wing.  The moment along the wingspan can be seen below in Figure C.3.

Figure C-3: Moment along the span of the wing.

EquationC.4:σxx=MyIzAfter calculating the moment on the wing, the normal force felt by the wing could be calculated using equation C.4.

As seen by the graph in Figure C.4, the highest magnitude of normal force, approximately 2750 psi, is felt at the sparcaps.

Figure C.4: Normal Stress vs. Height of spar cap.

Comparing this amount of force with the yield stress of spruce (8600psi), it can be seen that the structure of the wing is capable of withstanding the loads felt by it.

## Landing Gear

Correct sizing and placement of the landing gear is essential to the stability of the aircraft on the ground.  Guidelines for the layout of the conventional taildragger landing gear arrangement followed the steps given in Raymer’s Aircraft Design book.  With the aircraft stationary on the ground at the takeoff gross weight, the taildown angle would be about 10-15 deg.  Also, the center of gravity should fall between 16-25 deg back from the main wheel location.  Placement and sizing of the landing gear is essential to the stability of the aircraft on the ground.  Figure C.5 shows the method described above in placing and sizing the landing gear.

Figure C.5:  Figure 11.4 from Raymer’s Aircraft Design shows placement and sizing method for landing gear.

Equipment Layout

The equipment was placed inside the fuselage to accommodate the designated center of gravity.  Space was left for packing around each piece of equipment to prevent damage.  The equipment layout is shown along with a table showing the size and weight of each component below in Figure C.6 and Table C.1.

Figure C.6: A color-coded representation of the layout of the equipment

.

Table C.1:  Size and weight of each piece of equipment used in SID-5.

Equipment	  Volume (in3)		Weight (oz)

Gear box 	             3 x 1.5 x 1                     1.5

Motor		2.25 x 1.5                      7.5

Speed Controller  1.5 x 1.25 x 1                3.0

Receiver               1.75 x 1.25 x 0.75         1.0

Gyro                     1.5 x 1.25 x 1.25           3.5

Data Recorder      1.75 x 2.25 x 3.25         8.0

Battery (18)          2 x 1 x 1                        2.8

Servo (4)              1.5 x 1.25 x 0.75           2.0

Interface               1.25 x 3.5 x 5.75           7.0

Table C.2:  Estimated weight breakdown of SID-5.

11.0 (oz)

Fuselage

9.8 (oz)

Misc

9.5 (oz)

Tail

42.0 (oz)

Wing

2.0(oz)

Servo (4)

2.8(oz)

Cell weight (18)

1.0(oz)

Propeller

1.5(oz)

Gearbox

7.5(oz)

Motor

15.0(oz)

Tattletail8

3.5(oz)

Gyro

3.0(oz)

Speed controller

1.0(oz)

Receiver

D here

Appendix E: Performance

This appendix contains information on aircraft performance.  The following equations can be found in [Ref 5.1].  Turn radius and cruise speed were constraint values rather than calculated.   Actual performance data was calculated using yard markers on the football field and a stopwatch.

W = weight (lbs)

S = wing area (sqft)

rho = air density (slugs per cubic foot)

g = gravitational constant (feet per second squared)

T = thrust (lbf)

Tave = average thrust for all mission segments calculated by sizing code [App. G]

D = drag (lbf)

Gamma = climb angle (degrees)

Vmax = velocity (ft per second)

1) Takeoff Distance = (1.44*(W @ Takeoff)^2)/(rho*S*g*T)

2) Gamma = asin((T-D)/W)

3) Endurance = Battery Capacity/(Tave * Vcruise)

4) Stall speed = sqrt(W/(1/2*rho*S*CL))

Flight Card 1

| A/C NAME: | SID-5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| FLIGHT NUMBER | 1 |  |  |  |  |
| CREW CHIEF: | Pat Dempsey |  |  |  |  |
| GROUND CREW: | Keith Hout | Bridget Fitzpatrick | Heather Garber | JS Mok |  |
| PILOT | Dave Henady |  |  |  |  |
| DATE: | Saturday 18 NOV |  |  |  |  |
| TX FREQUENCY: | Channel 36 | ATM. PRESS. | 30.22 mm Hg | TEMP: | 30 F |
| FLIGHT ACTIVITY | First Flight |  |  |  |  |
| Location | Delphi Airport |  |  |  |  |
| Mission | Attempted First Flight |  |  |  |  |
| Comments | ~extremely windy |  |  |  |  |
|  | ~had problem keeping airplane from blowing away |  |  |  |  |
|  | ~sustained minor damage from being blown into parked car |  |  |  |  |
|  | ~need to add down trim to plane-Pilot |  |  |  |  |
|  | ~initially was too cold for motor to function, had to be brought in and warmed up |  |  |  |  |
|  | ~plane was predictable and handled well-Pilot |  |  |  |  |
| Duration | 3 minutes |  |  |  |  |
| Take off Distance | ~plane basically hopped into the air |  |  |  |  |
| Landing | ~no major problems |  |  |  |  |
| Battery Utilized | Ni-Cd |  |  |  |  |
| Damage Report | ~cracked rib |  |  |  |  |
| Repair Time | ~minor |  |  |  |  |

Flight Card 2

| A/C NAME: | SID 5 |  | DATE: | 19-Nov |
| --- | --- | --- | --- | --- |
| FLIGHT NUMBER(S) | 2 |  |  |  |
| CREW CHIEF: | Pat Dempsey |  | PILOTs: | Pat Dempsey |
|  |  |  |  | Dave Henady |
| GROUND CREW: | Keith Hout, Bridget Fitzpatrick, Heather Garber, J.S. Mok |  |  |  |
| TX FREQUENCY: | 36 | ATM. PRESS. | 30.30 mmHg | TEMP:46F |
| FLIGHT ACTIVITY | Second Flight |  |  |  |
| Location | Mollenkopf |  |  |  |
| Mission | Short hop | (P.D.) | A |  |
|  | Short hop and turn | (P.D.) | B |  |
|  | Endurance Mission | (D.H.) | C |  |
| Comments | A |  |  |  |
|  | Fine take off and landing needed some power. |  |  |  |
|  | B |  |  |  |
|  | Fine take off, very stable turn and hard landing due to not |  |  |  |
|  | adding enough power. |  |  |  |
|  | C |  |  |  |
|  | Very smooth take off,tunred and flew very nicely. |  |  |  |
|  | Very predictable flying and stable. |  |  |  |
|  | Very Maneuverable. |  |  |  |
|  | A lot of power. |  |  |  |
| Duration | A | B | C |  |
|  | less than 15 sec. | About 20 sec. | Appr. 14 min. 30 sec. |  |
| Flight Speed | A | B | C |  |
|  | 25 ft/s. | 25 ft/s. | 27~30 ft/s. |  |
| Take Off Distance | A | B | C |  |
|  | 15 yards. | 15 yards. | 20 yards. |  |
| Landing Distance | A | B | C |  |
|  | 10 yards. | 2 yards | 40~50 yards. |  |
| Battery | Ni-MH |  |  |  |
| Damage Report | A | No damage. |  |  |
|  | B | Separated firewall of motor. |  |  |
|  | C | No damage. |  |  |
| Repair Time | A | N/A |  |  |
|  | B | 5 min. |  |  |
|  | C | N/A |  | Card# |

Flight Card 3

| A/C NAME: | SID-5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| FLIGHT NUMBER | 3 |  |  |  |  |
| PILOT: | Dave Henady |  |  |  |  |
| DATE: | Sunday 19 NOV |  |  |  |  |
| TX FREQUENCY | Channel 36 | ATM PRESSURE | 30.30 mm Hg | TEMP | 46 F |
| FLIGHT ACTIVITY | Flap Test |  |  |  |  |
| Location | Mollenkopf |  |  |  |  |
| Mission | Use of flaps for indoor flight |  |  |  |  |
| Comments | ~Flight started out well |  |  |  |  |
|  | ~Adding flaps on landing is not the best idea |  |  |  |  |
|  | ~Plane was stopped by student to keep from hitting wall |  |  |  |  |
|  | ~adding flaps pitched plane up, and had to add more down elevator |  |  |  |  |
|  | ~Group heart attack occurred shortly after crash |  |  |  |  |
|  | ~it was demonstrated that feedback gain can be used |  |  |  |  |
| Duration | 5 minutes |  |  |  |  |
| Take Off Distance | 20 yards |  |  |  |  |
| Landing | ~Plane was caught in ground effect. Would not land. |  |  |  |  |
|  | ~Touched down late and then hit student |  |  |  |  |
| Battery Utilized | Ni-Cd |  |  |  |  |
| Damage Report | ~crushed leading edge on left side of bottom wing |  |  |  |  |
|  | ~crushed leading edge on majority of upper wing |  |  |  |  |
|  | ~about 15 broken ribs |  |  |  |  |
|  | ~broken elevator |  |  |  |  |
|  | ~major damage to group morale |  |  |  |  |
| Repair Time | ~substantial, about 90 man hours |  |  |  |  |

Flight Card 4

| A/C NAME: | SID-5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| FLIGHT NUMBER | 4 |  |  |  |  |
| PILOT | Dave Henady |  |  |  |  |
| CREW CHIEF: | Pat Dempsey |  |  |  |  |
| GROUND CREW: | Keith Hout | Bridget Fitzpatrick | Heather Garber | JS Mok |  |
| DATE: | Tuesday 21 NOV |  |  |  |  |
| TX FREQUENCY: | Channel 36 | ATM. PRESS. | 30.33 mm Hg | TEMP: | 46 F |
| FLIGHT ACTIVITY | W and W/O Feedback Gain |  |  |  |  |
| Location | Mollenkopf |  |  |  |  |
| Mission | demonstrate a/c flight both w and w/o feedback gain |  |  |  |  |
| Comments | ~Very predictable |  |  |  |  |
|  | ~gyro dampened out oscillations |  |  |  |  |
|  | ~made the controls more sluggish |  |  |  |  |
| Duration | 5 minutes |  |  |  |  |
| Take off Distance | 15 yards |  |  |  |  |
| Landing | 30 yards. No problems |  |  |  |  |
| Battery Utilized | Ni-Cd |  |  |  |  |
| Damage Report | Happily None to Report |  |  |  |  |
| Repair Time | Nonexistent |  |  |  |  |

Flight Card 5

| A/C NAME: | SID-5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| FLIGHT NUMBER | 5 |  |  |  |  |
| PILOT | Sean Henady |  |  |  |  |
| DATE: | Tuesday 21 NOV |  |  |  |  |
| TX FREQUENCY: | Channel 36 | ATM. PRESS. | 30.33 mm Hg | TEMP: | 46 F |
| FLIGHT ACTIVITY | Endurance+Manuevers |  |  |  |  |
| Location | Mollenkopf |  |  |  |  |
| Mission | perform the a/c mission of 12 minutes |  |  |  |  |
| Comments | ~did not make 12 minutes because performed a lot of maneuvers |  |  |  |  |
|  | ~flew nicely |  |  |  |  |
|  | ~did four circles hands off |  |  |  |  |
|  | ~pilot enjoyed flying this plane |  |  |  |  |
| Duration | 10:58 minutes |  |  |  |  |
| Take off Distance | 20 yards |  |  |  |  |
| Landing | 30 yards. No problems |  |  |  |  |
| Battery Utilized | Ni-MH |  |  |  |  |
| Damage Report | Happily None to Report |  |  |  |  |
| Repair Time | Nonexistent |  |  |  |  |

Flight Card 6

| A/C NAME: |  |  | DATE: | 21-Nov |
| --- | --- | --- | --- | --- |
| FLIGHT NUMBER(S) | 6 |  |  |  |
| PILOT: | Sean Henady |  |  |  |
| TX FREQUENCY: | 36 | ATM. PRESS. | 30.33 mmHg | TEMP:46F |
| FLIGHT ACTIVITY | Sixth Flight |  |  |  |
| Location | Mollenkopf |  |  |  |
| Mission | High Performance Test |  |  |  |
| Comments | Short take off with full throttle setting. |  |  |  |
|  | Tight turn with 6 yard radius. |  |  |  |
|  | Demonstrated nice roll rate. |  |  |  |
|  | Successful stall turns. |  |  |  |
|  | Tested the minimal stall speed with power on and off. |  |  |  |
|  | With power off stall speed was less than appr. 20 ft/s. |  |  |  |
|  | With power on stall speed was less than appr.15 ft/s. |  |  |  |
|  | Achieved maximum speed in Mollenkopf was appr.100 ft/s. |  |  |  |
| Duration | Appr. 8 min. |  |  |  |
| Flight Speed | Between 10 and 68 mph. |  |  |  |
| Take Off Distance | 8 yards ( 24 ft.) |  |  |  |
| Landing Distance | 10 yards |  |  |  |
| Battery | Ni-Cd. |  |  |  |
| Damage Report | No Damage. |  |  |  |
| Repair Time | N/A. |  |  |  |

Appendix F: Cost Analysis

This appendix contains information on cost analysis.  All pricing information was accurate as of 31 October 2000.  A detailed price breakdown of SID-5 is shown in Table F.1.  The total of all parts used on SID-5 at the time of flight is denoted as “used parts” while “kit price” denotes the cost of materials normally included in a commercial radio controlled airplane kit.  Standard kits include wood, hardware (screws, control horns, etc.), plans, and building instructions.  Kits do not include control systems, propulsion systems, glue or covering material (MonoKote).  Since SID-5 is a scratch built airplane there are no building instructions, thus no instructions will be furnished with the kit leaving the exact construction method up to the builder.

Table F.1:  Price breakdown of SID-5.

Appendix G: Design Codes

1. Matlab sizing code

2. Matlab aerodynamics code

3. Matlab structures code

Size

Aero

Struct