% Matlab Code to perform propeller analysis in a manner similar to gold.f

echo off

format compact

clear

disp(' ')

disp('Start new run')

disp('Propeller Analysis using Goldsteins Classical Vortex Theory')

disp('  This code works for two bladed propellers only.')

disp(' ')

% Input constants

Din=12;      % propeller diameter (inches)

Pin=6;      % pitch of the propeller (inches)

RPM=4000;    % RPM of propeller

Vmph=0 ;    % airspeed in mph

rho=.002378; % air density (slug/ft**2) (sea level)

%rho=.002065; % air density (slug/ft**2) at 4750 feet

% Pin gives geometric angle to the flat part of the

%   rear of the propeller

aoldeg=-6;   % angle of zero lift of the propeller (degrees)

%  measured from mean chord line (typically negative)

beta0deg=.5; % angle from flat part of the prop to mean chord line

a=2*pi;      % lift curve slope of propeller

Cd0=.00655;  % 2-d minimum drag coefficient

k=.01;       % Cd = Cd0+k*Cl*Cl

B=2 ;        % number of blades (2 for standard type propeller)

% input nondimensional properties at each radial location

% cR=c/R, x=r/R

x=[.3,.35,.4,.45,.5,.55,.6,.65,.7,.75,.8,.85,.9,.95,1.];

cR=.09*ones(size(x));

% END OF INPUTS

% Display inputs section

disp(['Diameter Din= ',num2str(Din),' inches'])

disp(['Pitch Pin= ',num2str(Pin),' inches'])

disp(['Angle of zero lift aoldeg= ',num2str(aoldeg),' degrees'])

disp(['Angle Flat side to mean chord beta0deg= ',num2str(beta0deg),' degrees'])

disp(['RPM= ',num2str(RPM),' rpm'])

disp(['2-d Lift curve slope a= ',num2str(a),' per rad'])

disp(['2-d Min drag coef Cd0= ',num2str(Cd0)])

disp(['k from 2-d drag polar= ',num2str(k)])

disp(['Number of blades B= ',num2str(B)])

disp(['air density rho= ',num2str(rho),' slug/ft^3'])

disp(['Airspeed Vmph= ',num2str(Vmph),' mph'])

% derived constants

V=Vmph*88/60; % airspeed in ft/sec

D=Din/12;     % Diameter in feet

R=D/2;        % Radius in feet

n=RPM/60;     % propeller frequency (rev/sec) or (hz)

omega=2*pi*n; % frequency of revolution of the propeller (rad/sec)

lamda=V/(omega*R);

r2d=180/pi;

Vt=omega*R;   % tip velocity (ft/sec)

J=V/(n*D);    % advance ratio

% Output scalar constants

disp(['Airspeed V= ',num2str(V),' ft/sec'])

disp(['Propeller Diameter D= ',num2str(D),' feet'])

disp(['Propeller Radius R= ',num2str(R),' feet'])

disp(['propeller RPS n= ',num2str(n),' hertz'])

disp(['omega= ',num2str(omega),' rad/sec'])

disp(['lamda= ',num2str(lamda)])

disp(['r2d= ',num2str(r2d),' deg/rad'])

disp(['Tip speed Vt= ',num2str(Vt),' ft/sec'])

disp(['Advance Ratio J= ',num2str(J)])

disp(' ')

%derived section constants

c=R*cR;    % chord in feet

cin=c*12;  % chord in inches

disp(' ')

disp('x in r/R and is nondimensional, cR=c/R, cin in the chord in inches')

echo on

disp([x',      cR',       cin'])

echo off

disp(' ')

beta1=atan(((Pin/Din)/pi)./x);

beta=beta1+(beta0deg-aoldeg)/r2d;

sigma=B*c/(pi*R);

r=x*R;

Vr=Vt*sqrt(x.*x+lamda*lamda);

phi=atan(lamda./x);

WtVt=.02*ones(size(c));  %initial guess

nr=length(c);

nr1=nr-1;

aiold=zeros(size(c));

for ii=1:40

WaVt(1:nr1)=.5*(-lamda+sqrt(lamda*lamda+4*WtVt(1:nr1).*(x(1:nr1)-WtVt(1:nr1))));

ai(1:nr1)=atan(WtVt(1:nr1)./WaVt(1:nr1))-phi(1:nr1);

%ai(1:nr1)=atan((V+WaVt(1:nr1)*Vt)./(omega*r(1:nr1)-WtVt(1:nr1)*Vt))-phi(1:nr1);

e=sum(abs(ai(1:nr1)-aiold(1:nr1)));

iter=['Loop index= ',num2str(ii),'  error= ',num2str(e)];

disp(iter)

if e<.0001 ; break; end

aiold(1:nr1)=ai(1:nr1);

Cl(1:nr1)=a*(beta(1:nr1)-ai(1:nr1)-phi(1:nr1));

VeVt(1:nr1)=sqrt((lamda+WaVt(1:nr1)).^2+(x(1:nr1)-WtVt(1:nr1)).^2);

gamma(1:nr1)=.5*c(1:nr1).*Cl(1:nr1).*VeVt(1:nr1)*Vt;

sinphialp(1:nr1)=sin(phi(1:nr1)+ai(1:nr1));

kappa(1:nr1)=kappa2(x(1:nr1),sinphialp(1:nr1));

WtVt(1:nr1)=B*gamma(1:nr1)./(4*pi*Vt*r(1:nr1).*kappa(1:nr1));

end

Cl(nr)=0;

ai(nr)=beta(nr)-phi(nr);

VrVt=sqrt(lamda*lamda+1);

WaVt(nr)=VrVt*sin(ai(nr))*cos(ai(nr)+phi(nr));

WtVt(nr)=VrVt*sin(ai(nr))*sin(ai(nr)+phi(nr));

VeVt(nr)=sqrt((lamda+WaVt(nr))^2+(x(nr)-WtVt(nr))^2);

kappa(nr)=0;

Cd=Cd0+k*Cl.*Cl;

ZT=(pi/8)*(J*J+pi*pi*(x.*x)).*sigma;

ZP=pi*ZT.*x;

dCTdx=ZT.*(Cl.*cos(phi+ai)-Cd.*sin(phi+ai));

dCPdx=ZP.*(Cl.*sin(phi+ai)+Cd.*cos(phi+ai));

% Overall propeller performance

CT=trapi(dCTdx,x);

CP=trapi(dCPdx,x);

eta=CT*J/CP;

T=CT*rho*n^2*D^4;

P=CP*rho*n^3*D^5;

HP=P/550;

Pwatt=1.356*P;

torque=P/omega;

Clmax=max(Cl);

Toz=T*16;

PestWatts=1.31*D^4*(Pin/12)*(RPM/1000)^3;

% The above approximate formula works for

% Top Flite, Zinger and Master Airscrews reasonably well.

% For Rev Up props subract .5 in from the pitch.

% For APC props use constant 1.11 instead of 1.31.

% For thin carbon fiber folding props use 1.18 instead of 1.31.

% Ref: Electric Motor Handbook, by Robert J. Boucher,

%    AstroFlight, Inc.

echo on

dat=[ x',     beta',     phi',     kappa',     ai',     WtVt']

disp(' ')

dat2=[x',     WaVt',     VeVt',    Cl',     dCTdx',    dCPdx']

echo off

disp(' ')

disp(['Advance Ratio J= ',num2str(J)])

disp(['Thrust Coefficient CT= ',num2str(CT)])

disp(['Power Coefficient CP= ',num2str(CP)])

disp(['Propeller efficiency eta= ',num2str(eta)])

disp(['Speed V= ',num2str(V),' ft/sec'])

disp(['RPM= ',num2str(RPM),' rpm'])

disp(['Thrust T= ',num2str(T),' pounds'])

disp(['Thrust Toz= ',num2str(Toz),' ounces'])

disp(['Power used P= ',num2str(P),' ft*lbf/sec'])

disp(['Horsepower used HP= ',num2str(HP),' HP'])

disp(['Power used Pwatt= ',num2str(P),' watts'])

disp(['Torque used Q= ',num2str(torque),' ft*lbf'])

disp(['Clmax= ',num2str(Clmax)])

disp(' ')

disp(['Estimated power used, PestWatts= ',num2str(PestWatts),' watts, Ref: Boucher'])

disp(' ')

subplot(211)

plot(x,dCTdx)

z=axis;

axis([0,1,0,z(4)])

xlabel('nondimensional radial location')

ylabel('dCTdx')

subplot(212)

plot(x,dCPdx)

z=axis;

axis([0,1,0,z(4)])

xlabel('nondimensional radial location')

ylabel('dCPdx')

disp(' For model aircraft propellers this code underestimates')

disp(' the power required. The underestimation is worse for RPM>10,000')

disp(' where it may underestimate by a factor of .5')

disp(' For RPM <10,000 the factor is about .75')