CE 503 HOMEWORK 5

Assigned 1 Friday, 1 November,  Due Wednesday 20 November

- Look at the postcard image in “postcrd1a.tif” and stereo digitize some control points (building roof corners, etc.) from the purdue model being used in the map compilation project.

- Put those control points into “control.dat” in the format: point number, East, North, Height (ascii text).

- Observe the corresponding points on the postcard image (in adobe photoshop for example) and record in “obs.dat” in the format: point number, column, row ( or equivalently point number, photoshop-x, photoshop-y).

- Use these two files as input to “dlt.m” to obtain approximations for omega, phi, kappa, XL,YL,ZL, focal length, etc.

- Now use “resect.m” to perform space resection (nonlinear, iterative model) to refine the exterior orientation with a sensor model approach.

- Initial approximations go into “cam.inp” in the format: omega (radians), phi, kappa, XL,YL,ZL,x0,y0,f (ascii text)

- Assuming that you get a converged and reasonable solution, then do a “manual” version of self-calibration by systematically varying the interior orientation parameters f, x0, y0 to get the best solution (smallest residuals).

- Document all steps, draw some conclusions, and write a small report on the this effort.